3U PIONEER ACADEMY SDN BHD

NATIONAL OCCUPATIONAL SKILLS STANDARD (NOSS)

IT-020-3:2013

COMPUTER SYSTEM OPERATION

(Operasi Sistem Komputer)

LEVEL 3

26 March 2026

TABLE OF CONTENTS

# STANDARD PRACTICE

Standard Practice

National Occupational Skills Standard (NOSS) for Computer System Operation Level 3

Code:** IT-020-3:2013

1. Introduction

The ever-changing world fuelled largely by the continuing advancement in computer technology requires organizations to be well-equipped to remain current and, to a certain extent, relevant. Businesses from multi-national corporations to home-based enterprises have to adopt current technologies to be competitive. Government agencies and institutions too, need to stay at the top of the game in order to provide fast, reliable and accurate services to the people.

This climate creates job opportunities in computer system operation to support the industry growth. The computer system operation personnel or known as Computer System Technician by most of the organisations is the first level of the computer technical support classification. It is distinguished from the higher level Computer Specialist class by the latter's more advanced and specialized responsibility to provide support.

As part of the computer system operation team, computer system technicians are much sought after by many organizations that depends strongly on reliable working condition of computer system. Generally, computer system technicians perform computer and peripheral set-up, installation and tests of computer hardware and software, server installation and maintenance, network connectivity set-up, as well as maintenance and basic repair of equipment. In addition, they may also be responsible to keep records of software and equipment, take details of user problems by phone, in person or via e-mail, and identify and solve computer problems.

As the computer system technician is defined as a skilled-job area, its career advancement very much depends on individual experience and performance. This NOSS highlights the core competencies that can be acquired by a computer system technician.

### Pre-requisite

The candidate must complete lower secondary school and must have correct colour vision to pursue this course.

2. Occupational Structure

The Existing Occupational Structure of Computer System Technician is illustrated in Figure 1. The NOSS development expert panels have proposed a new occupational structure as shown in Figure 2, while proposed Occupational Area Structure is illustrated in Figure 3.

This job area is a single tier which specialises in computer system operation. The expert panels have agreed that the entry level for Computer System Operation is at Level 3 due to their nature of work where generally they work by following instructions and job assignment schedules that is prepared by a superior. The personnel perform a significant range of varied work activities in a variety of context, which most of the tasks are complex and non-routine.

There is a significant career path for this job title. An experienced Computer System Technician with advanced training may become IT Executive, the level 4 personnel.

### Figure 1: Existing Occupational Structure (MDeC, 2012)

Industry: ICT | Sub-Sector:** Service Management and Delivery

| Level | Infrastructure Support | Help Desk / Call Centre |
| --- | --- | --- |
| L5 | Associate Systems Specialist Infrastructure | Associate Help Desk Support Specialist |
| L4 | IT Executive | Help Desk Executive |
| L3 | -- | -- |
| L2 | -- | -- |
| L1 | -- | -- |

### Figure 2: Proposed Occupational Structure (NOSS Expert Panels, 2012)

| Level | Infrastructure Support | Application Systems Dev | Help Desk |
| --- | --- | --- | --- |
| L5 | Associate Systems Specialist, Computer Network Manager, IT Manager | System Analyst | Associate Help Desk Support Specialist |
| L4 | IT Executive, Computer Network Executive | Analyst Programmer | Help Desk Executive |
| L3 | Computer Network Technician, Computer System Technician | Lead Programmer | -- |
| L2 | -- | -- | -- |
| L1 | -- | -- | -- |

### Figure 3: Proposed Occupational Area Structure (NOSS Expert Panels, 2012)

| Level | Infrastructure Support | Application Dev | Help Desk |
| --- | --- | --- | --- |
| L5 | Computer Network Management, Computer System Management | System Implementation & Administrator | Associate Help Desk Support Specialist |
| L4 | Computer Network Administration, Computer System Administration | System Module Development | Help Desk Executive |
| L3 | Computer Network Service, Computer System Operation | System Application Support | -- |
| L2 | -- | -- | -- |
| L1 | -- | -- | -- |

3. Description of Competency Level

The NOSS is developed for various occupational areas. Candidates for certification must be assessed and trained at certain levels to substantiate competencies. Below is a guideline of each NOSS Level as defined by the Department of Skills Development, Ministry of Human Resources, Malaysia.

| Level | Title | Description |
| --- | --- | --- |
| L1 | SKM (Operation/Production) | Routine and predictable work activities. |
| L2 | SKM (Operation/Production) | Varied work in multiple contexts; some non-routine, requiring individual responsibility. |
| L3 | SKM (Supervisory) | Complex, non-routine work; considerable autonomy; control/guidance of others often required. |
| L4 | DKM (Executive) | Complex technical/professional work; substantial personal responsibility; resource allocation and supervision of others. |
| L5 | DLKM (Managerial) | Fundamental principles and complex techniques across unpredictable contexts; significant responsibility for others and substantial resources; accountable for analysis, diagnosis, planning, execution, and evaluation. |

4. Malaysian Skill Certification

Candidates who have attended and successfully completed each competency shall be awarded Certificate of Attendance on that competency. Those who have attended and successfully completed all the core competencies and fulfilled Malaysian Skill certification requirements shall receive Sijil Kemahiran Malaysia (Malaysia Skills Certificate) in Computer System Operation at Level 3.

5. Job Competencies

Computer System Technician is competent in performing the following core competencies:

1. Computer System Set-up

2. Computer System Maintenance

3. Computer System Repair

4. Server Installation

5. Server Maintenance

6. Computer Network Connectivity Set-up

7. Mobile Device Configuration

6. Working Condition

Computer system technicians generally work in offices, computer rooms and at their clients' workplaces. The job requires them to constantly move around, either within the office or to other business premises. Those who are responsible to repair computer and peripheral usually work indoors in repair shops or data processing centres. Those who travel to users must carry their tools and testing equipment with them. As some computer malfunctions can now be diagnosed by remote access, this lessens travel requirements for them.

Computer system technicians generally work 40 hours a week at normal office hours, but depending on the nature of the business, they may have to work beyond the normal hours including on weekends, or be on call to make emergency repairs or to meet project deadlines. Many computer system technicians especially in large corporations work on rotating shifts. Conditions may be stressful as they are usually asked to fix faults quickly.

Computer system technicians need to have basic knowledge of computer hardware, software and networks, up-to-date information about the latest developments in ICT, problem-solving ability and good communication skills for dealing with clients or peers. They need to be able to work both independently and as part of a team and be able to work well under pressure. In addition, they also need to have good hand-eye-coordination and must be able to lift heavy boxes and computer hardware.

On-the-job trainings may be offered from time to time, so a computer system technician must be prepared to continue learning new computer skills and are willing to attend a wide range of courses to update their knowledge and techniques.

7. Employment Prospect

Computer system technicians are employed by a range of organizations including:

- Government departments and agencies

- Institutions of higher learning

- Private corporations

- Banks

- Private companies that provide computer, database and network services to clients

- Telecommunication companies

- Many other private organizations

A growing number of computer system technicians are hired on a temporary or contract basis. Many of these individuals are self-employed, working independently as contractors or consultants.

8. Training, Industrial/Professional Recognition, Other Qualifications and Advancement

### 8.1 Industrial Recognition / Professional Qualification

There are a few professional certifications that recognise IT personnel such as:

- The Computing Technology Industry Association (CompTIA)

- Cisco Networking Academy

- Linux Professional Institute

- SANS Institute

- EC-Council

### 8.2 Other Prominent Qualifications Recognised (Malaysia or International)

Training is likely to be on the job, learning how systems work and how to deal with clients and gather the information required identifying the problem. By working with more experienced technical support officers, new employees can learn the most common problems and possible solutions. Larger organisations may offer external training courses or in-house training opportunities.

Technologies change rapidly and it is an essential part of the Computer System Technician to remain up to date with any development in hardware and software, as well as being aware of previous versions that may still be in operation.

The CompTIA A+ certification is a professional international certification for computer technicians. Officially, CompTIA A+ certification is a vendor neutral certification that covers numerous technologies and operating systems from various vendors.

### 8.3 Types of Occupation for Career Advancement

Career advancement for computer operation level 3 personnel locally and internationally is enormous. Among them are:

- Computer Specialist

- Computer Technologies

- IT Sales Professional

- Technical Sales Engineer

### 8.4 Related Industries

Computer system personnel are employed in every sector of the economy, private and public sector.

9. Sources of Additional Information

### 9.1 Local Organisations

- MOSTI (NITC Secretariat) -- 62662 Putrajaya -- www.mosti.gov.my

- MCMC -- 63000 Cyberjaya -- www.skmm.gov.my

- PIKOM (National ICT Association of Malaysia) -- 46350 Petaling Jaya -- www.pikom.org.my

- MDeC -- 63000 Cyberjaya -- www.mdec.my

- MAMPU -- 62502 Putrajaya -- www.mampu.gov.my

- Ministry of Multimedia, Culture & Heritage -- 50610 Kuala Lumpur -- www.kpkk.gov.my

### 9.2 International Organisations

- American Society of Information Science and Technology -- www.asis.org

- Association for Computing Machinery (ACM) -- www.acm.org

- CISCO Malaysia -- www.cisco.com

- EC-Council Asia Pacific -- www.eccouncil.org

- IEEE Computer Society -- www.computer.org

- Institute for Certification of Computing Professionals -- www.iccp.org

- Linux Professional Institute -- www.lpi.org

- Network Professional Association -- www.npanet.org

- CompTIA -- www.comptia.org

10. Approval Date

The National Skills Development Board (NSDB), Ministry of Human Resources has agreed and endorsed this Standard.

11. Acknowledgement

The Director General of DSD would like to extend his gratitude to the organisations and individuals who have been involved in developing this standard.

### NOSS Development Committee Members -- Computer System Operation Level 3

Expert Panels:

| No. | Name | Role / Organisation |
| --- | --- | --- |
| 1 | Ahmad Syauqi Bin Mahmud | Senior Tech Executive, CMCA Sdn Bhd (MSC-Status) |
| 2 | Amir Bin Jamalluddin | IT Security Officer, MCIS Zurish Sdn. Bhd. |
| 3 | Fairus Zaki Bin Omar | Systems Engineer, Premisnet System (M) Sdn. Bhd. (MSC-Status) |
| 4 | Izzudin Bin Ismail | Business Information Strategy Manager, ICT Zone Sdn. Bhd. (MSC-Status) |
| 5 | Juraidawati Binti Arbain | Senior Lecturer Computer Sciences, Universiti Industri Selangor (UNISEL) |
| 6 | Kasful Anuar Bin Kassim | Infra Analyst, ASTRO Television Network System Sdn. Bhd. (MSC-Status) |
| 7 | Muhamad Dzukfakar Bin Zaiton | IT Executive, GLOMAC |
| 8 | Noor Azmi Bin Bahaldin | IT Support, iPerintis Sdn. Bhd (MSC-Status) |
| 9 | Nur Adriany Suraya Binti Yahya | Account Manager, GLOMAC Sdn. Bhd. |
| 10 | Zawahir Bin Mohd Wazir | IT Officer, Majlis Sukan Negeri Selangor |
| 11 | Zulfadli Bin Md Zain | Wintel Engineer, Kunng Tech Sdn Bhd (MSC-Status) |

Facilitator:

| No. | Name | Organisation |
| --- | --- | --- |
| 12 | Jaiyah Binti Shahbudin | Worldbay Solution Sdn. Bhd. |

Co-Facilitator:

| No. | Name | Organisation |
| --- | --- | --- |
| 13 | Isvaran A/L P. Ramasamy | Worldbay Solution Sdn. Bhd. |

Glossary

| Abbreviation | Full Form |
| --- | --- |
| ADGP | Advanced Graphics Port |
| AES | Advance Encryption Standard |
| BIOS | Basic Input Output System |
| CD | Compact Disc |
| CD-ROM | Compact Disc Read Only Memory |
| CPU | Central Processing Unit |
| DHCP | Dynamic Host Configuration Protocol |
| DMZ | Demilitarised Zone |
| DNS | Domain Name Server |
| DoS | Denial of Service |
| DRAM | Dynamic Random Access Memory |
| ECD | Error Correcting Code |
| EMS | Environment Monitoring System |
| FTP | File Transfer Protocol |
| ICMP | Internet Control Message Protocol |
| IP | Internet Protocol |
| IPS | Intrusion Prevention System |
| LAN | Local Area Network |
| LCD | Liquid Crystal Display |
| MAN | Metropolitan Area Network |
| Mbps | Megabit per second |
| MB | Megabyte |
| MD5 | Message Digest number 5 |
| NAC | Network Access Control |
| NAT | Network Address Translation |
| NDA | Non-Disclosure Agreement |
| NIC | Network Interface Card |
| NMS | Network Monitoring System |
| NRPE | Nagios Remote Plug-in Executor |
| OEM | Original Equipment Manufacturer |
| OS | Operating System |
| PC | Personal Computer |
| PCI | Personal Computer Interconnect |
| PENTEST | Penetration Test |
| RAID | Redundant Array of Inexpensive Disks |
| RAM | Random Access Memory |
| RADIUS | Remote Authentication Dial In User Service |
| ROM | Read Only Memory |
| SAT | Secure Ada Target |
| SCSI | Small Computer System Interface |
| SDRAM | Synchronous Dynamic Random Access Memory |
| SOP | Standard Operating Procedure |
| SRAM | Static Random Access Memory |
| TCP | Transmission Control Protocol |
| TKIP | Temporary Key Integrity Protocol |
| UAT | User Acceptance Test |
| USB | Universal Serial Bus |
| VPN | Virtual Private Network |
| WAN | Wide Area Network |
| WEP | Wired Equivalent Privacy |
| WPA | Wi-Fi Protected Access |

Competency Profile Chart (CPC)

| Field | Value |
| --- | --- |
| Sector | Information Communication Technology |
| Sub Sector | Infrastructure Support |
| Job Area | Computer System Operation |
| Job Level | Three (3) |
| Job Area Code | IT-020-3:2013 |

### Core Competencies

| Competency Unit | Code |
| --- | --- |
| Computer System Set-up | IT-020-3:2013-C01 |
| Computer System Maintenance | IT-020-3:2013-C02 |
| Computer System Repair | IT-020-3:2013-C03 |
| Server Installation | IT-020-3:2013-C04 |
| Server Maintenance | IT-020-3:2013-C05 |
| Computer Network Connectivity Set-up | IT-020-3:2013-C06 |
| Mobile Device Configuration | IT-020-3:2013-C07 |

# CONTACT HOUR DISTRIBUTION

Program: IT-020-3:2013 Computer System Operation (Operasi Sistem Komputer)

Level: L3

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION | IT-020-3:2013 COMPUTER SYSTEM OPERATION |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | Computer System Operation (Operasi Sistem Komputer) | Computer System Operation (Operasi Sistem Komputer) |
| TOTAL CONTACT HOURS | 1200 | 1200 |
| KNOWLEDGE 30% / PERFORMANCE 70% | 360 hrs / 840 hrs | 360 hrs / 840 hrs |
| REFERENCE | 10_CoCU Contact hour_Computer L3 · Kitchen LV2 C01 | PAGE: Appendix |

| CoCu | Competency unit | % | Total hrs | Knowledge 30% | Performance 70% |
| --- | --- | --- | --- | --- | --- |
| 1 | Computer System Set-up | 25% | 300 | 90 | 210 |
| 2 | Computer System Maintenance | 10% | 120 | 36 | 84 |
| 3 | Computer System Repair | 15% | 180 | 54 | 126 |
| 4 | Server Installation | 20% | 240 | 72 | 168 |
| 5 | Server Maintenance | 15% | 180 | 54 | 126 |
| 6 | Computer Network Connectivity Set-up | 10% | 120 | 36 | 84 |
| 7 | Mobile Device Configuration | 5% | 60 | 18 | 42 |
| Total |  | 100% | 1200 | 360 | 840 |

| % | Hrs | WORK ACTIVITIES | Knowledge 30% | Performance 70% | Total |
| --- | --- | --- | --- | --- | --- |
| CoCu 1: Computer System Set-up |  |  | 90.0 | 210.0 | 300.0 |
| 10% | 30 | Analyse job request/change order | 9.0 | 21.0 | 30.0 |
| 15% | 45 | Prepare computer set-up tools, hardware parts and software | 13.5 | 31.5 | 45.0 |
| 30% | 90 | Set-up computer hardware | 27.0 | 63.0 | 90.0 |
| 20% | 60 | Carry out computer software installation | 18.0 | 42.0 | 60.0 |
| 10% | 30 | Set-up computer peripherals | 9.0 | 21.0 | 30.0 |
| 10% | 30 | Carry out unit functionality test | 9.0 | 21.0 | 30.0 |
| 5% | 15 | Prepare computer system set-up report | 4.5 | 10.5 | 15.0 |
| CoCu 2: Computer System Maintenance |  |  | 36.0 | 84.0 | 120.0 |
| 15% | 18 | Identify computer maintenance requirements | 5.4 | 12.6 | 18.0 |
| 40% | 48 | Carry out computer scheduled preventive maintenance | 14.4 | 33.6 | 48.0 |
| 40% | 48 | Perform computer corrective maintenance | 14.4 | 33.6 | 48.0 |
| 5% | 6 | Prepare computer maintenance report | 1.8 | 4.2 | 6.0 |
| CoCu 3: Computer System Repair |  |  | 54.0 | 126.0 | 180.0 |
| 10% | 18 | Assess computer repair job order/change request | 5.4 | 12.6 | 18.0 |
| 30% | 54 | Carry out online trouble shooting | 16.2 | 37.8 | 54.0 |
| 50% | 90 | Perform on-site repair | 27.0 | 63.0 | 90.0 |
| 10% | 18 | Prepare computer status report | 5.4 | 12.6 | 18.0 |
| CoCu 4–7 | *(see individual CoCu docs for full work-activity breakdown)* |  | 252.0 | 588.0 | 600.0 |
| TOTAL |  |  | 360.0 | 840.0 | 1200.0 |

| Tool / Equipment | Description | Category |
| --- | --- | --- |
| Tools and parts | Standard toolkit: screwdrivers, anti-static strap, USB installer. Parts from standard inventory; one | Other |
| Report | Set-up report with serial numbers, OS version, and user sign-off. | Other |
| Screwdriver set (Phillips, flat, Torx) | The most used tools in computer set-up. Used to open the chassis, secure drives and expansion cards, | Hand Tools |
| Multimeter | Used to check power supply voltages (e.g. +3.3 V, +5 V, +12 V), continuity, and basic electrical saf | Testing Equipment |
| Cable tester / LAN tester | Used to verify network cable wiring (e.g. straight-through, crossover) and connectivity. Confirms th | Testing Equipment |
| Anti-static mat and wrist strap | Reduces electrostatic discharge (ESD) when handling motherboards, RAM, and other sensitive component | ESD Protection |
| USB bootable media / optical drive | Used to load the operating system and drivers during software installation. The installer is prepare | Installation Media |
| Label maker or tags | Used to label cables, ports, and assets for identification and reporting. Labels help during trouble | Labeling Equipment |
| 30% | 90 | Other |
| 20% | 60 | Other |
| 10% | 30 | Other |
| 5% | 15 | Other |
| CPU (Central Processing Unit) | The processor that executes instructions. It must match the socket type of the motherboard (e.g. LGA | Hardware Components |
| RAM (Random Access Memory) | Temporary storage for running programs and data. Types include DDR5; capacity and speed must be comp | Hardware Components |
| Motherboard | The main board that connects the CPU, RAM, storage, and peripherals. Form factor (e.g. ATX, micro-AT | Hardware Components |
| Storage (HDD / SSD / NVMe) | HDD (hard disk drive) is used for bulk storage at lower cost; SSD and NVMe offer much faster boot an | Hardware Components |
| Power supply unit (PSU) | Converts AC from the wall to DC and supplies the correct voltages to the motherboard and components. | Hardware Components |
| Peripherals | Monitor, keyboard, mouse, printer, and other devices. Connected via USB, HDMI or DisplayPort, or leg | Peripherals |
| Operating system (OS) | Windows, Linux, or other OS as per job request. Installation media (USB or disc) and a valid licence | Software |
| Drivers | Device drivers for the motherboard (chipset, LAN, audio), GPU, and peripherals. Often supplied by th | Hardware Components |
| Applications | Office, browser, antivirus, and other applications as specified in the change order or standard buil | Software |
| System utilities and firmware | Built-in tools (disk management, device manager, task manager) and BIOS/UEFI firmware used to config | Software |
| Set-up computer hardware | Understand installation procedures and hardware compatibility | Other |
| Carry out computer software installation | Know procedures and troubleshooting techniques | Other |
| Set-up computer peripherals | Understand installation procedures and hardware compatibility | Peripherals |
| Carry out unit functionality test | Know procedures and troubleshooting techniques | Other |
| Prepare computer system set-up report | Know tools, parts, and materials needed | Other |
| Compressed air / blower | Used to remove dust from vents, heatsinks, fans, and inside the chassis without touching components. | Other |
| Soft brush (anti-static) | Used with compressed air to loosen dust from heatsinks and fan blades. A small brush helps reach tig | ESD Protection |
| Thermal paste / compound | Applied between the CPU and heatsink to improve heat transfer. Degrades over time; reapply when rese | Other |
| Isopropyl alcohol (IPA) and lint-free cloth | Used to clean old thermal paste from the CPU and heatsink before applying new paste. High concentrat | Other |
| Screwdriver set and anti-static strap | Same as in set-up: open chassis, remove heatsink, and secure components. Anti-static strap is worn a | Hand Tools |
| Temperature and diagnostics software | Used to check CPU, GPU, and disk temperatures and to view SMART data and event logs. Tools such as H | Other |
| Dust and airflow management | Dust blocks vents and heatsinks and causes overheating. The system should be powered off and unplugg | Other |
| Thermal paste and heatsink | The compound between the CPU and heatsink that improves heat transfer. It degrades over time and sho | Other |
| Fans and cooling | CPU fan, case fans, and GPU fan keep components within operating temperature. Monthly checks ensure | Other |
| BIOS / UEFI firmware | Firmware that controls boot and hardware settings. Updates can fix compatibility and security issues | Software |
| Operating system updates | Security and cumulative updates applied as per organisational policy (e.g. weekly or monthly). Sched | Software |
| Antivirus definitions | Virus definition files must be kept current. Verify definitions are up to date during each scheduled | Other |
| Event log and diagnostics | The OS and hardware keep logs (e.g. Windows Event Viewer, SMART for disks) that record errors and wa | Other |
| Backup and restore tools | Software for creating and verifying backups of user data and system state before corrective maintena | Other |
| Tasks performed | List of tasks (cleaning, parts replaced, updates applied, tests run). A detailed task listing enable | Other |
| Parts and materials | Part numbers and quantities used; none if preventive only. Recording parts supports inventory manage | ESD Protection |
| Sign-off | Technician name and date; user or supervisor acceptance if required. Sign-off provides accountabilit | Other |
| Phone / communication tool | Used to gather symptoms from the user, guide them step-by-step (e.g. restart, safe mode, check cable | Other |
| Remote desktop / remote support tool | Used when the user can boot and connect to the network: view their screen, run diagnostics (event lo | Other |
| Knowledge base / diagnostic scripts | Used to look up error codes, beep codes, and known fixes; run scripted checks (e.g. disk SMART, memo | Other |
| Spare parts and USB installer | Known-good RAM, PSU, or disk for swap testing; USB bootable media for OS reinstall or recovery. Veri | Installation Media |
| Diagnostic software | Memory test (e.g. Memtest86), disk SMART tools, event log viewer, and temperature monitors. Run afte | Other |
| POST (Power-On Self-Test) | Firmware checks run at boot that test RAM, drives, keyboard, and other devices. Beep codes or on-scr | Other |
| RAM modules | Temporary storage for running programs. Faulty RAM causes no POST, random crashes, or blue screens. | Hardware Components |
| Storage drives (HDD / SSD / NVMe) | Failed or failing drives cause no boot, data loss, or slow performance. Check SMART data for warning | Hardware Components |
| Safe mode / recovery environment | The OS loads with minimal drivers and services. Used to isolate software versus hardware faults and | Other |
| Event log and minidump analysis | Windows Event Viewer (Application and System logs) and minidump files record errors, warnings, and c | Other |
| Replacement parts compatibility | Use parts that match the system's specifications: same socket, form factor, interface, and supported | Other |
| Backup verification | Before performing destructive repairs (OS reinstall, disk replacement), verify that a current backup | Other |
| Test result | Outcome of post-repair testing (POST, boot, diagnostics, stress test if required). Documenting test | Other |
| Recommendation | Follow-up actions such as monitoring, user training, further repair, or disposal. Recommendations gu | Other |
| Rack rails and cage nuts | Used to mount rack servers into the server rack. Rails are specific to the server model and rack typ | Other |
| Cable management arms and label maker | Cable management arms attach to the rear of the server and organise power, network, and management c | Network Equipment |
| USB bootable media | Used to load the server operating system (Windows Server, Linux) and drivers during software install | Installation Media |
| Network cable tester | Used to verify network cable wiring and connectivity between the server NIC and the switch port. Con | Testing Equipment |
| Server chassis / form factor | Rack servers (1U, 2U, etc.) are mounted in a standard 19-inch rack with rails and require proper coo | Other |
| Redundant power supply (PSU) | Dual or hot-swap PSUs provide continuous power if one unit fails. Both PSUs are connected to differe | Hardware Components |
| RAID controller and disk array | Redundant array of independent disks for capacity and/or data redundancy (e.g. RAID 1 for mirroring, | Hardware Components |
| Network interfaces (NICs) | One or more network interface cards providing connectivity to the LAN, management network, or storag | Other |
| Out-of-band management (iLO / iDRAC) | Remote management interfaces such as HP iLO or Dell iDRAC that provide console access, power control | Other |
| Server operating system | Windows Server or Linux (e.g. CentOS, Ubuntu Server, RHEL) installed from USB or network boot as per | Software |
| Roles and features | Server roles such as file server, DNS, DHCP, Active Directory, Hyper-V, or web server are installed | Other |
| Patches and updates | Security and cumulative updates applied after the base OS installation to bring the server to curren | Other |
| Backup and recovery configuration | Backup software and schedule configured as part of the server installation to protect data from the | Other |
| Software configuration | OS version, installed roles and features, key applications, and patch level. Documenting the softwar | Other |
| Network settings | IP addresses, subnet mask, VLAN, gateway, and DNS servers. Accurate network records prevent IP confl | Other |
| Carry out software installation | Know procedures and troubleshooting techniques | Other |
| Perform server functionality test | Know testing procedures and acceptance criteria | Other |
| Prepare server installation set-up report | Know tools, parts, and materials needed | Other |
| Vacuum cleaner (anti-static / ESD-safe) | Used to remove dust from server vents, heatsinks, fans, and internal components. Dust build-up restr | ESD Protection |
| Cable ties and labels | Used to organise and label server cables (power, network, SAN, KVM) for identification and airflow m | Network Equipment |
| Backup software and media | Used to perform and verify server data backups before maintenance. Backup media includes external st | Installation Media |
| Server management / monitoring software | Remote management tools (e.g. iDRAC, iLO, IPMI) and monitoring dashboards used to check server utili | Peripherals |
| Server chassis and rack mount | The physical enclosure that houses all server components. Rack-mounted servers (1U, 2U, 4U) slide in | Other |
| Server power supply unit (PSU) | Provides power to all server components. Enterprise servers typically have redundant PSUs (e.g. 1+1) | Hardware Components |
| Server fans and cooling system | Maintain operating temperature inside the server. Servers have multiple fans with redundancy; a fail | Other |
| Hard disk drives (HDD / SSD) in RAID array | Server storage configured in RAID (Redundant Array of Independent Disks) for performance and data pr | Hardware Components |
| Uninterruptible Power Supply (UPS) | Provides battery backup power during mains power failure, allowing graceful server shutdown or conti | Hardware Components |
| Storage Area Network (SAN) devices | External storage connected to the server via fibre channel or iSCSI. During maintenance, check SAN c | Hardware Components |
| Server LED status indicators | Physical LEDs on the front and rear panels that show system health at a glance. Key indicators inclu | Other |
| Server utilisation monitoring | Checking CPU usage, memory usage (physical RAM, virtual, shared), hard disk space (used, free, total | Peripherals |
| Server data backup | Performing full backups (complete copy of all data) and incremental backups (only data changed since | Other |
| Server system logs | Three main log types: Security Log (login attempts, access events, policy changes), Application Log | Other |
| Server antivirus management | Maintaining antivirus software by checking: antivirus update log (pattern/definition currency), anti | Other |
| Operating System patches and updates | Applying security bulletins and critical patches to the server OS. Check patch availability through | Software |
| Critical OS patch fails to install | Patch conflicts with installed software, insufficient disk space, or service dependency not met | Other |
| Antivirus definitions outdated / scan not running | Automatic update disabled, network proxy blocking update server, or licence expired | Other |
| UPS battery low or failed self-test | Battery degradation over time, excessive load, or environmental temperature too high | Other |
| Crimping tool and RJ45 connectors | Used to terminate UTP cable with RJ45 connectors. The crimping tool presses the connector contacts o | Other |
| Punch-down tool | Used to terminate cable wires onto patch panels and keystone jacks (faceplates). The tool seats the | Other |
| Managed network switch | The central device that connects all network drops. Managed switches allow VLAN configuration, port | Other |
| Router / gateway | Connects the local network to the WAN or internet. Configured with the gateway IP address, NAT, DHCP | Other |
| Wireless access point (AP) | Provides Wi-Fi connectivity for wireless devices. Configured with SSID, security protocol (WPA2/WPA3 | Other |
| Label maker and cable tags | Used to label every cable run, patch panel port, faceplate, and switch port with a unique identifier | Network Equipment |
| Network Interface Card (NIC) | The adapter that connects a computer or peripheral to the network. NICs may be built into the mother | Other |
| UTP cable (Cat5e / Cat6 / Cat6a) | Unshielded twisted pair cable used for Ethernet LAN connections. Cat5e supports up to 1 Gbps at 100 | Network Equipment |
| Fibre optic cable (single-mode / multi-mode) | Used for long-distance runs or high-speed backbone connections between switches, floors, or building | Network Equipment |
| Patch panel | A passive device mounted in the network rack that terminates all cable runs from wall faceplates. Ea | Other |
| RJ45 faceplate and keystone jack | Mounted on the wall at each network drop location. The keystone jack terminates the cable from the p | Other |
| Main Distribution Frame (MDF) and rack | The central location where all network cabling terminates and active equipment (switches, routers, p | Hardware Components |
| IP addressing and subnetting | Assigning IP addresses to devices using a static scheme or DHCP server. The subnet mask defines the | Other |
| VLAN configuration | Virtual LANs segment the physical network into logical groups (e.g. by department, function, or secu | Network Equipment |
| Network connectivity testing tools | Standard tools for verifying connectivity: ping (tests reachability to gateway, DNS, and externa | Other |
| Network security configuration | Basic security settings applied during network set-up: setting the switch management password, disab | Other |
| Mobile Device Management (MDM) software | Enterprise software (e.g. Microsoft Intune, VMware Workspace ONE, Jamf) used to remotely configure, | Other |
| SIM ejector tool and SIM cards | The SIM ejector tool opens the SIM tray on smartphones and tablets. SIM cards (nano, micro, or eSIM) | Other |
| USB cable and charging adapter | Used to connect the mobile device to a computer for initial setup, data transfer, firmware update, o | Network Equipment |
| Wi-Fi network access | Required during initial setup to download updates, enrol in MDM, install apps, and configure cloud a | Other |
| Diagnostic and testing apps | Tools used to verify device functionality after configuration: Wi-Fi analyser (signal strength and c | Other |
| Smartphone (Android / iOS) | A mobile phone with a full operating system capable of running corporate applications. Key specifica | Other |
| Tablet (Android / iOS / Windows) | A larger-screen mobile device used for field work, meetings, presentations, or kiosk applications. C | Other |
| Laptop (portable computer) | A portable computer running Windows, macOS, or Linux. Configuration includes OS setup, network (Wi-F | Other |
| Connectivity interfaces | The connection types available on mobile devices: Wi-Fi (802.11ac/ax for wireless LAN), **Cellul | Other |
| Mobile device security features | Built-in security capabilities: screen lock (PIN, pattern, fingerprint, face recognition), **dev | Other |
| Network and connectivity configuration | Setting up Wi-Fi (SSID, security key, 802.1X if enterprise), cellular APN, Bluetooth pairing, and VP | Other |
| Email and collaboration setup | Configuring corporate email using Exchange ActiveSync, IMAP, or POP as specified in the job order. S | Other |
| Mobile application management | Installing, configuring, and updating required applications per the job order. Apps may be deployed | Software |
| Security policy enforcement | Applying the organisational security policy to the device: enable device encryption, set screen lock | Other |

| No. | Document | Code | PAGE |
| --- | --- | --- | --- |
| 1 | 01_CoCu-1-Computer-System-Set-up | IT-020-3:2013 - CoCu 1 / P(1/7) | 1 - 38 |
| 2 | 02_CoCu-2-Computer-System-Maintenance | IT-020-3:2013 - CoCu 2 / P(2/7) | 39 - 52 |
| 3 | 03_CoCu-3-Computer-System-Repair | IT-020-3:2013 - CoCu 3 / P(3/7) | 53 - 72 |
| 4 | 04_CoCu-4-Server-Installation | IT-020-3:2013 - CoCu 4 / P(4/7) | 73 - 112 |
| 5 | 05_CoCu-5-Server-Maintenance | IT-020-3:2013 - CoCu 5 / P(5/7) | 113 - 132 |
| 6 | 06_CoCu-6-Computer-Network-Connectivity-Set-up | IT-020-3:2013 - CoCu 6 / P(6/7) | 133 - 144 |
| 7 | 07_CoCu-7-Mobile-Device-Configuration | IT-020-3:2013 - CoCu 7 / P(7/7) | 145 - 154 |

# CoCu 1: Computer System Set-up (300 hrs)

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION |  |
| --- | --- | --- |
| LEVEL | L3 |  |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 1: Computer System Set-up |  |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB REQUEST/CHANGE ORDER · PREPARE COMPUTER SET-UP TOOLS, HARDWARE PARTS AND SOFTWARE · SET-UP COMPUTER HARDWARE · CARRY OUT COMPUTER SOFTWARE INSTALLATION · SET-UP COMPUTER PERIPHERALS · CARRY OUT UNIT FUNCTIONALITY TEST · PREPARE COMPUTER SYSTEM SET-UP REPORT |  |
| NO. CODE | IT-020-3:2013 - CoCu 1 / P(1/7) |  |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 1: Computer System Set-up (Persediaan Sistem Komputer) | CoCu 1: Computer System Set-up (Persediaan Sistem Komputer) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB REQUEST/CHANGE ORDER · PREPARE COMPUTER SET-UP TOOLS, HARDWARE PARTS AND SOFTWARE · SET-UP COMPUTER HARDWARE · CARRY OUT COMPUTER SOFTWARE INSTALLATION · SET-UP COMPUTER PERIPHERALS · CARRY OUT UNIT FUNCTIONALITY TEST · PREPARE COMPUTER SYSTEM SET-UP REPORT | ANALYSE JOB REQUEST/CHANGE ORDER · PREPARE COMPUTER SET-UP TOOLS, HARDWARE PARTS AND SOFTWARE · SET-UP COMPUTER HARDWARE · CARRY OUT COMPUTER SOFTWARE INSTALLATION · SET-UP COMPUTER PERIPHERALS · CARRY OUT UNIT FUNCTIONALITY TEST · PREPARE COMPUTER SYSTEM SET-UP REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 1 / P(1/7) | PAGE: 1 - 22 |

| SET-UP CONTEXT | DESKTOP / STANDARD BUILD | WORKSTATION / CUSTOM BUILD |
| --- | --- | --- |
| Job request | Standard office build: single PC, standard OS and applications, one user. Job request specifies model, OS version, and software list. | Custom or workstation build: high-end CPU/GPU, multiple drives, special software or dual-boot. Change request may specify hardware list, RAID, and software versions. |
| Tools and parts | Standard toolkit: screwdrivers, anti-static strap, USB installer. Parts from standard inventory; one system unit, monitor, keyboard, mouse. | Extended toolkit: multimeter, cable tester, thermal paste. Parts ordered per spec; multiple drives, expansion cards, or special peripherals. |
| Report | Set-up report with serial numbers, OS version, and user sign-off. | Set-up report with full hardware list, RAID config, software versions, and acceptance test result. |

| Type of Tools | Description |
| --- | --- |
| Screwdriver set (Phillips, flat, Torx) | The most used tools in computer set-up. Used to open the chassis, secure drives and expansion cards, and assemble or disassemble computer components. Phillips and flat heads are for general screws; Torx for manufacturer-specific screws on some brands. An anti-static wrist strap is recommended when handling boards to avoid electrostatic discharge (ESD) that can damage components. |
| Multimeter | Used to check power supply voltages (e.g. +3.3 V, +5 V, +12 V), continuity, and basic electrical safety before and after set-up. Ensures the PSU is within tolerance and that there are no short circuits before powering on the system. |
| Cable tester / LAN tester | Used to verify network cable wiring (e.g. straight-through, crossover) and connectivity. Confirms that each wire is correctly terminated and that the cable can carry signal. Essential when laying or repairing network cables. |
| Anti-static mat and wrist strap | Reduces electrostatic discharge (ESD) when handling motherboards, RAM, and other sensitive components. The wrist strap is worn and connected to a grounded point or the chassis; the mat provides a grounded surface for components. Never handle boards without grounding in dry environments. |
| USB bootable media / optical drive | Used to load the operating system and drivers during software installation. The installer is prepared on USB or disc according to the job request (e.g. Windows, Linux). Some systems require legacy boot or secure boot settings in BIOS/UEFI. |
| Label maker or tags | Used to label cables, ports, and assets for identification and reporting. Labels help during troubleshooting and when completing the set-up report with asset and serial information. |

| % | Hrs | Work activity | Knowledge 30% | Performance 70% | Total |
| --- | --- | --- | --- | --- | --- |
| 10% | 30 | Analyse job request/change order | 9.0 | 21.0 | 30.0 |
| 15% | 45 | Prepare computer set-up tools, computer hardware parts and computer software | 13.5 | 31.5 | 45.0 |
| 30% | 90 | Set-up computer hardware | 27.0 | 63.0 | 90.0 |
| 20% | 60 | Carry out computer software installation | 18.0 | 42.0 | 60.0 |
| 10% | 30 | Set-up computer peripherals | 9.0 | 21.0 | 30.0 |
| 10% | 30 | Carry out unit functionality test | 9.0 | 21.0 | 30.0 |
| 5% | 15 | Prepare computer system set-up report | 4.5 | 10.5 | 15.0 |
| 100% | 300 |  | 90.0 | 210.0 | 300.0 |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 1: Computer System Set-up (Persediaan Sistem Komputer) | CoCu 1: Computer System Set-up (Persediaan Sistem Komputer) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB REQUEST/CHANGE ORDER · PREPARE COMPUTER SET-UP TOOLS, HARDWARE PARTS AND SOFTWARE · SET-UP COMPUTER HARDWARE · CARRY OUT COMPUTER SOFTWARE INSTALLATION · SET-UP COMPUTER PERIPHERALS · CARRY OUT UNIT FUNCTIONALITY TEST · PREPARE COMPUTER SYSTEM SET-UP REPORT | ANALYSE JOB REQUEST/CHANGE ORDER · PREPARE COMPUTER SET-UP TOOLS, HARDWARE PARTS AND SOFTWARE · SET-UP COMPUTER HARDWARE · CARRY OUT COMPUTER SOFTWARE INSTALLATION · SET-UP COMPUTER PERIPHERALS · CARRY OUT UNIT FUNCTIONALITY TEST · PREPARE COMPUTER SYSTEM SET-UP REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 1 / P(1/7) | PAGE: 23 - 38 |

| Hardware Component | Description |
| --- | --- |
| CPU (Central Processing Unit) | The processor that executes instructions. It must match the socket type of the motherboard (e.g. LGA1700, AM5). Thermal paste and a heatsink or fan are required; the paste fills microscopic gaps between the CPU and heatsink for efficient heat transfer. Incorrect installation can cause overheating and permanent damage. |
| RAM (Random Access Memory) | Temporary storage for running programs and data. Types include DDR5; capacity and speed must be compatible with the motherboard and CPU. Modules are installed in the correct slots (often pairs for dual-channel). Not fully seated or incompatible RAM can cause no POST or random crashes. |
| Motherboard | The main board that connects the CPU, RAM, storage, and peripherals. Form factor (e.g. ATX, micro-ATX) determines case compatibility. The BIOS or UEFI firmware is used to set boot order, secure boot, and hardware options. All other components connect to the motherboard. |
| Storage (HDD / SSD / NVMe) | HDD (hard disk drive) is used for bulk storage at lower cost; SSD and NVMe offer much faster boot and application load. Interface (SATA, M.2 PCIe Gen5) must match the motherboard. NVMe Gen5 drives use M.2 PCIe Gen5 slots and often require a standoff and screw for mounting. |
| Power supply unit (PSU) | Converts AC from the wall to DC and supplies the correct voltages to the motherboard and components. Wattage must meet the total system requirement (CPU, GPU, drives). Cabling must match the motherboard (e.g. 24-pin, 8-pin CPU, PCIe for GPU). Modular PSUs allow unused cables to be left off for better airflow. |
| Peripherals | Monitor, keyboard, mouse, printer, and other devices. Connected via USB, HDMI or DisplayPort, or legacy ports as specified in the job request. Drivers and settings (e.g. resolution, refresh rate) are configured during software installation and set-up. |

RAID Level Comparison

| Software Type | Description |
| --- | --- |
| Operating system (OS) | Windows, Linux, or other OS as per job request. Installation media (USB or disc) and a valid licence or key are required. The installer partitions the drive, copies files, and configures the initial user and settings. Updates and drivers are applied after the base install. |
| Drivers | Device drivers for the motherboard (chipset, LAN, audio), GPU, and peripherals. Often supplied by the OS or from the manufacturer website. Missing or wrong drivers can cause devices not to work or to perform poorly. Install in the order recommended by the manufacturer (e.g. chipset first). |
| Applications | Office, browser, antivirus, and other applications as specified in the change order or standard build. May be installed from media, network share, or app store. Licence keys and configuration (e.g. default browser, updates) are applied according to organisational policy. |
| System utilities and firmware | Built-in tools (disk management, device manager, task manager) and BIOS/UEFI firmware used to configure, monitor, and troubleshoot the system after installation. Firmware updates may be needed for hardware compatibility. Utilities are used during set-up to verify partitions, check device status, and confirm system performance before user handover. |

| Common Fault During Set-up | Cause | Action |
| --- | --- | --- |
| No power / no POST | PSU not switched on, power cable loose, front-panel wires wrong, or PSU fault | Check power cable and switch; verify front-panel header; test with known-good PSU if needed |
| No display | Monitor not connected or wrong input; GPU not seated; RAM not seated | Check cable and input; reseat GPU and RAM; try onboard video if available |
| OS installer not booting | Boot order wrong; secure boot or legacy setting; corrupted USB | Set boot order in BIOS/UEFI; disable secure boot for some installers; recreate USB installer |
| Driver or device not working | Wrong or missing driver; driver not installed after OS | Install correct driver from manufacturer; run Windows Update or equivalent |

Learning Outcome Matrix

| Work Activity | Knowledge Outcome | Performance Outcome | Assessment Method | Evidence Required |
| --- | --- | --- | --- | --- |
| Analyse job request/change order | Identify and interpret requirements | Analyse requirements and extract key priorities | Written test on requirement analysis; oral questions on decision criteria | Completed analysis checklist; documented decision rationale |
| Prepare computer set-up tools, computer hardware parts and computer software | Know tools, parts, and materials needed | Gather and organize tools, parts, and materials | Practical observation of tool/material preparation; checklist verification | Photo/log of prepared toolkit; materials checklist |
| Set-up computer hardware | Understand installation procedures and hardware compatibility | Correctly assemble and install components according to specifications | Practical hands-on assembly; functionality test verification | Correctly assembled system; successful test results |
| Carry out computer software installation | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Set-up computer peripherals | Understand installation procedures and hardware compatibility | Correctly assemble and install components according to specifications | Practical hands-on assembly; functionality test verification | Correctly assembled system; successful test results |
| Carry out unit functionality test | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Prepare computer system set-up report | Know tools, parts, and materials needed | Gather and organize tools, parts, and materials | Practical observation of tool/material preparation; checklist verification | Photo/log of prepared toolkit; materials checklist |

Practical Exercises

Lab 1.1: Analysing and Planning a Computer Build

Equipment Required

Job request template, Customer specifications sheet, Hardware checklist, Parts inventory list, Scheduling tool

Procedures

- 1. Receive a simulated job request specifying CPU model, RAM capacity, storage type, and software requirements

- 2. Extract and verify all customer requirements from the job request

- 3. Cross-check hardware specifications against supplier inventory

- 4. Create a logical set-up sequence (order of installation steps)

- 5. Identify any missing parts or incompatibilities and document actions

- 6. Prepare a bill of materials and tools checklist

- 7. Document the analysis in a structured format for team review

Expected Outcome

Completed analysis showing identified requirements, compatibility verification, and set-up plan

Assessment Checklist

- Job request completely read and understood

- All hardware specifications extracted and verified

- Incompatibilities or gaps identified

- Set-up sequence documented in logical order

- Bill of materials prepared

- Analysis signed off for proceeding to set-up

Lab 1.2: Gathering and Organizing Set-up Materials

Equipment Required

Full technician toolkit, Hardware components, Anti-static mat and wrist strap, USB installer, Software media, Labelling equipment

Procedures

- 1. Gather all tools from the workshop: screwdrivers, multimeter, cable tester, anti-static strap, thermal paste

- 2. Retrieve required hardware components from inventory: CPU, RAM, storage, motherboard, power supply, peripherals

- 3. Prepare OS and driver media (USB installers, disc if required)

- 4. Set up anti-static mat in the work area and verify grounding

- 5. Organize all materials in a logical sequence for installation

- 6. Create a checklist confirming all items are present and functional

- 7. Label cables, connectors, and components for easy identification during installation

Expected Outcome

Organized workspace with all tools and materials ready, verified checklist, and labelled components

Assessment Checklist

- Toolkit is complete and tools are functional

- All hardware components are present and undamaged

- Software media is prepared and bootable

- Anti-static precautions are in place

- Workspace is organized and clear

- All items verified on equipment checklist

Lab 1.3: Hardware Assembly and Installation

Equipment Required

Computer case, Motherboard, CPU with heatsink, RAM, Storage drives, Power supply, Expansion cards, Cable management supplies

Procedures

- 1. Install the motherboard in the case, using correct standoffs and screws

- 2. Install the CPU with thermal paste and cooler, following manufacturer specifications

- 3. Install RAM modules in correct slots (verify dual-channel configuration if applicable)

- 4. Install storage devices (HDD/SSD/NVMe Gen5) in appropriate bays and slots

- 5. Connect all power connectors: 24-pin motherboard, 8-pin CPU, PCIe for GPU if present

- 6. Connect SATA, SSD, or NVMe Gen5 data cables (if using SATA, SSD, or NVMe Gen5 drives) or verify NVMe Gen5 seating

- 7. Organize and secure cables to avoid obstruction of airflow

- 8. Verify all components are properly seated and secured

- 9. Do not power on yet—prepare for BIOS checks

Expected Outcome

Fully assembled computer with all components correctly installed and cables organized

Assessment Checklist

- Motherboard is properly seated and secured

- CPU is installed with thermal paste and cooler attached

- RAM is installed in correct slots and fully seated

- Storage devices are installed and connected

- All power connectors are connected and secure

- Cables are organized and do not block airflow

- No loose screws or components remain

Lab 1.4: Operating System and Driver Installation

Equipment Required

Assembled computer from Lab 1.3, OS USB installer, Driver media/downloads, Monitor and peripherals

Procedures

- 1. Connect monitor, keyboard, and mouse; power on the system

- 2. Enter BIOS/UEFI to verify hardware detection (check CPU, RAM, storage)

- 3. Set boot order to USB and enable/disable secure boot as needed for OS installer

- 4. Boot from USB installer and follow OS installation prompts

- 5. Partition the storage drive according to job requirements

- 6. Complete base OS installation and initial configuration

- 7. Install chipset drivers from manufacturer support site or media

- 8. Install GPU, network, audio, and other device drivers in correct order

- 9. Apply Windows updates or OS patches

- 10. Verify all devices are recognized in Device Manager and no unknown devices remain

Expected Outcome

Working computer with OS installed and all drivers recognized; Device Manager shows no errors

Assessment Checklist

- OS installed successfully and boots without errors

- All hardware detected in BIOS and OS

- All device drivers installed (no unknown devices)

- Network and audio working

- No driver errors in Device Manager

- System stable for initial testing

Lab 1.5: Peripheral Connection and Configuration

Equipment Required

Installed computer from Lab 1.4, Monitor, Keyboard, Mouse, Printer (optional), USB peripherals

Procedures

- 1. Verify monitor is connected with correct cable (HDMI, DisplayPort, DVI)

- 2. Set monitor resolution and refresh rate in OS display settings to match hardware capability

- 3. Connect keyboard and mouse via USB and verify device detection

- 4. Install printer driver if a printer is part of the job request

- 5. Connect printer via USB or network and verify it appears in Devices and Printers

- 6. Test each peripheral: type with keyboard, move mouse, send test print if applicable

- 7. Configure peripheral settings (keyboard layout, mouse sensitivity, printer defaults)

Expected Outcome

All peripherals connected, recognized, and functioning correctly

Assessment Checklist

- Monitor connected and resolution correct

- Keyboard responds and is recognized

- Mouse responds and is recognized

- Printer (if applicable) installed and ready

- All peripherals visible in Device Manager with no errors

Lab 1.6: System Functionality and Stability Testing

Equipment Required

Fully configured system from Lab 1.5, System utilities (Windows, Linux built-in tools), Multimeter (optional)

Procedures

- 1. Power on the system and observe boot sequence for any errors or warnings

- 2. Log in and verify user account creation

- 3. Run Device Manager and check for any unknown or error devices

- 4. Use system utilities to check disk status: Run Check Disk (chkdsk) or equivalent

- 5. Monitor CPU and RAM usage during normal operations using Task Manager or top command

- 6. Test network connectivity by opening a browser and visiting a website

- 7. Copy a test file to storage and verify read/write speeds are acceptable

- 8. Launch one or two applications (office, browser) specified in the job and verify they open correctly

- 9. Stress-test the system briefly (e.g., run antivirus scan or CPU-intensive tool for 5 minutes)

- 10. Check temperatures using BIOS monitoring or system utilities; ensure CPU/GPU temps are in normal range

- 11. Document test results on the functionality test checklist

Expected Outcome

All system tests pass; system is stable, all devices respond, and no errors are logged

Assessment Checklist

- System boots without errors

- User account is functional

- All hardware is recognized with no unknown devices

- Disk check completed with no errors

- CPU and RAM are operating within expected parameters

- Network connectivity confirmed

- Applications launch and respond normally

- System remains stable under load

- CPU and GPU temperatures are within safe range

Lab 1.7: Documentation and Set-up Report Completion

Equipment Required

Tested computer, Set-up report template, Camera or photo capture device, Pen/signature area

Procedures

- 1. Gather serial numbers and product keys from all hardware components

- 2. Record installed OS version, edition, and licence key (if applicable)

- 3. Document all installed drivers and applications with versions

- 4. Record BIOS/UEFI settings (boot order, secure boot, XMP, TPM status) that differ from defaults

- 5. Capture a screenshot of Device Manager showing all recognized hardware

- 6. Take photos of hardware configuration and cable organization for future reference

- 7. Record RAM configuration (total capacity, speed, dual-channel status)

- 8. Record storage configuration (drive types, total capacity, partition layout)

- 9. Document any deviations from the original job request or changes made during set-up

- 10. Complete the set-up checklist with sign-off from a senior technician

- 11. Prepare handover notes for the user (user guide, support contact information)

Expected Outcome

Complete set-up report with all hardware details, software versions, test results, and user sign-off

Assessment Checklist

- All hardware serial numbers and specs recorded

- OS version and licence recorded

- All drivers and applications documented with versions

- Non-default BIOS settings recorded

- Hardware photos and Device Manager screenshot attached

- System configuration diagram or notes prepared

- Any deviations from job request explained

- Technician sign-off obtained

- User handover notes prepared

Employability Skills

This CoCu develops the following employability skills through the work activities:

- Activity: Prepare computer system set-up report

- Mapping: Write clear technical documentation; prepare reports showing hardware configurations, software versions, and test results for user handover and quality assurance

- Activities: Analyse job request/change order, Prepare computer set-up tools and hardware

- Mapping: Coordinate with help desk and other technicians; clarify user requirements from job requests; work together when sourcing parts and tools

- Activities: Carry out computer software installation, Carry out unit functionality test

- Mapping: Diagnose and resolve installation failures, driver issues, and hardware compatibility problems; troubleshoot boot failures and hardware conflicts

- Activities: Set-up computer hardware, Set-up computer peripherals

- Mapping: Work independently to assemble and configure systems; take personal responsibility for checking component compatibility and installation accuracy

- Activities: Analyse job request/change order, Prepare computer set-up tools and hardware

- Mapping: Plan the logical sequence of setup tasks; organize tools and components before starting; follow correct installation order (chipset drivers, then device drivers)

- All activities, especially those involving electrical equipment

- Mapping: Follow ESD protocols and electrical safety practices; manage time to meet deadlines; maintain organized workspace and documentation

- All activities requiring hands-on work with hardware and software

- Mapping: Use multimeters, cable testers, and diagnostic tools; configure BIOS/UEFI settings; install and configure OS and drivers; troubleshoot using system utilities

- All activities; especially relevant for new hardware platforms and OS versions

- Mapping: Stay current with new hardware standards and driver updates; learn from installation failures; apply lessons to improve future procedures

Attitude, Safety and Environmental

- Electrostatic Discharge (ESD) Protection: Always wear an anti-static wrist strap and work on a grounded surface when handling motherboards, RAM, CPU, and expansion cards. Even small static charges can permanently damage sensitive components.

- Electrical Safety: When testing with a multimeter or working near power supplies, ensure the power is disconnected and use proper testing techniques. Follow all electrical safety guidelines and never work on live circuits.

- Thermal Compound Handling: Apply thermal paste correctly between CPU and heatsink to prevent overheating and damage. Use only the recommended amount and allow proper curing time before powering on.

- Safe Cable Management: Ensure network and power cables are properly routed to avoid tripping hazards, kinks, and damage. Label cables clearly for future troubleshooting and maintenance.

- E-waste Management: Properly dispose of defective hardware components, circuit boards, and packaging materials through authorised e-waste recycling centres. Never dispose of electronics in regular trash or landfill.

- Energy Efficiency: Configure systems to use power-saving modes and efficient power supplies. Avoid unnecessary idle time and follow organisational energy conservation policies.

- Punctuality and Reliability: Complete system set-up work on schedule to meet user handover deadlines and support business operations.

- Integrity in Documentation: Prepare accurate and complete set-up reports with all hardware details, software versions, and test results. Honest documentation supports future troubleshooting and maintains system integrity.

- Teamwork and Support: Collaborate with help desk staff and other technicians; share knowledge about system configurations and troubleshooting techniques to improve overall technical capability.

- Attention to Detail: Verify every connection, configuration setting, and test result to ensure systems are correctly set up and function as intended.

References

# CoCu 2: Computer System Maintenance (120 hrs)

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION |  |
| --- | --- | --- |
| LEVEL | L3 |  |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 2: Computer System Maintenance |  |
| NO. AND WORK ACTIVITY STATEMENT | IDENTIFY COMPUTER MAINTENANCE REQUIREMENTS · CARRY OUT COMPUTER SCHEDULED PREVENTIVE MAINTENANCE · PERFORM COMPUTER CORRECTIVE MAINTENANCE · PREPARE COMPUTER MAINTENANCE REPORT |  |
| NO. CODE | IT-020-3:2013 - CoCu 2 / P(2/7) |  |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 2: Computer System Maintenance (Penyelenggaraan Sistem Komputer) | CoCu 2: Computer System Maintenance (Penyelenggaraan Sistem Komputer) |
| NO. AND WORK ACTIVITY STATEMENT | IDENTIFY COMPUTER MAINTENANCE REQUIREMENTS · CARRY OUT COMPUTER SCHEDULED PREVENTIVE MAINTENANCE · PERFORM COMPUTER CORRECTIVE MAINTENANCE · PREPARE COMPUTER MAINTENANCE REPORT | IDENTIFY COMPUTER MAINTENANCE REQUIREMENTS · CARRY OUT COMPUTER SCHEDULED PREVENTIVE MAINTENANCE · PERFORM COMPUTER CORRECTIVE MAINTENANCE · PREPARE COMPUTER MAINTENANCE REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 2 / P(2/7) | PAGE: 39 - 48 |

| SET-UP CONTEXT | PREVENTIVE (SCHEDULED) MAINTENANCE | CORRECTIVE MAINTENANCE |
| --- | --- | --- |
| Job request | Scheduled maintenance task from the maintenance calendar or asset register. Specifies systems due for routine cleaning, inspection, update, and backup verification. Frequency may be monthly, quarterly, or as per organisational policy. | Unscheduled task triggered by a user fault report, event log alert, or failed hardware. The job order describes the fault symptoms, affected system, and priority. Corrective work begins after preventive checks or when a new fault is reported. |
| Tools and parts | Compressed air, soft brush, thermal paste, isopropyl alcohol, lint-free cloth, screwdriver set, anti-static strap, and temperature/diagnostics software. No replacement parts expected unless inspection reveals a fault. | Same cleaning tools plus spare parts (fan, RAM, disk, PSU), USB bootable media, and diagnostic software. Parts are drawn from inventory based on the diagnosed fault. |
| Report | Maintenance report listing date, asset ID, tasks performed (cleaning, updates, backup check), findings, and sign-off. No parts used unless a fault was found during inspection. | Maintenance report listing date, asset ID, fault description, corrective actions taken, parts replaced (with serial numbers), test result, and sign-off. |

| Type of Tools | Description |
| --- | --- |
| Compressed air / blower | Used to remove dust from vents, heatsinks, fans, and inside the chassis without touching components. Hold fans still while blowing to avoid overspinning and damaging bearings. Use short bursts and do not use near eyes or mouth. In dusty environments, schedule cleaning more often. |
| Soft brush (anti-static) | Used with compressed air to loosen dust from heatsinks and fan blades. A small brush helps reach tight spaces. Anti-static brushes reduce risk of ESD when working near boards. Never use a vacuum directly on the motherboard as suction can generate static and damage components. |
| Thermal paste / compound | Applied between the CPU and heatsink to improve heat transfer. Degrades over time; reapply when reseating the heatsink or after several years. Use a small pea-sized amount in the centre; spread evenly or let pressure distribute. Excess paste can cause mess and poor contact; clean old paste with isopropyl alcohol before reapplying. |
| Isopropyl alcohol (IPA) and lint-free cloth | Used to clean old thermal paste from the CPU and heatsink before applying new paste. High concentration (90%+) evaporates quickly and leaves no residue. Wipe gently; allow to dry before applying new paste and reseating the heatsink. |
| Screwdriver set and anti-static strap | Same as in set-up: open chassis, remove heatsink, and secure components. Anti-static strap is worn and connected to a grounded point when handling RAM, boards, or CPU to avoid ESD damage during corrective maintenance. |
| Temperature and diagnostics software | Used to check CPU, GPU, and disk temperatures and to view SMART data and event logs. Tools such as HWiNFO, SpeedFan, and Windows Event Viewer help identify overheating or failing hardware before failure and support evidence for maintenance reports. |

| % | Hrs | Work activity | Knowledge 30% | Performance 70% | Total |
| --- | --- | --- | --- | --- | --- |
| 15% | 18 | Identify computer maintenance requirements | 5.4 | 12.6 | 18.0 |
| 40% | 48 | Carry out computer scheduled preventive maintenance | 14.4 | 33.6 | 48.0 |
| 40% | 48 | Perform computer corrective maintenance | 14.4 | 33.6 | 48.0 |
| 5% | 6 | Prepare computer maintenance report | 1.8 | 4.2 | 6.0 |
| 100% | 120 |  | 36.0 | 84.0 | 120.0 |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 2: Computer System Maintenance (Penyelenggaraan Sistem Komputer) | CoCu 2: Computer System Maintenance (Penyelenggaraan Sistem Komputer) |
| NO. AND WORK ACTIVITY STATEMENT | IDENTIFY COMPUTER MAINTENANCE REQUIREMENTS · CARRY OUT COMPUTER SCHEDULED PREVENTIVE MAINTENANCE · PERFORM COMPUTER CORRECTIVE MAINTENANCE · PREPARE COMPUTER MAINTENANCE REPORT | IDENTIFY COMPUTER MAINTENANCE REQUIREMENTS · CARRY OUT COMPUTER SCHEDULED PREVENTIVE MAINTENANCE · PERFORM COMPUTER CORRECTIVE MAINTENANCE · PREPARE COMPUTER MAINTENANCE REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 2 / P(2/7) | PAGE: 49 - 52 |

| Hardware Component | Description |
| --- | --- |
| Dust and airflow management | Dust blocks vents and heatsinks and causes overheating. The system should be powered off and unplugged before cleaning. Use compressed air and a soft brush; avoid vacuum directly on boards to reduce static risk. Clean fans, vents, and heatsinks according to the maintenance schedule (monthly in dusty environments, quarterly otherwise). Proper airflow depends on unobstructed vents and correctly routed cables inside the chassis. |
| Thermal paste and heatsink | The compound between the CPU and heatsink that improves heat transfer. It degrades over time and should be reapplied when reseating the heatsink or after several years of use. To replace: power off and unplug, disconnect CPU fan cable, remove heatsink, wipe old paste from CPU and heatsink with isopropyl alcohol and lint-free cloth, apply a small pea-sized amount of new paste to the centre of the CPU, reseat heatsink firmly and evenly, reconnect fan cable, and verify temperatures after power on. |
| Fans and cooling | CPU fan, case fans, and GPU fan keep components within operating temperature. Monthly checks ensure fans spin freely and are not noisy. Worn bearings cause noise and reduced airflow; replace the fan if noise persists after cleaning. Use BIOS/UEFI or temperature software to verify temperatures are within specification. |
| BIOS / UEFI firmware | Firmware that controls boot and hardware settings. Updates can fix compatibility and security issues; follow the manufacturer procedure and back up current settings before updating. A failed update can make the system unbootable. Check for updates during scheduled maintenance if the manufacturer has released a security or stability fix. |

| Software Component | Description |
| --- | --- |
| Operating system updates | Security and cumulative updates applied as per organisational policy (e.g. weekly or monthly). Schedule outside peak hours to minimise disruption. Verify that updates install correctly and that the system boots normally after reboot. Roll back updates if they cause instability. |
| Antivirus definitions | Virus definition files must be kept current. Verify definitions are up to date during each scheduled maintenance. Scan results and quarantine actions are recorded in the maintenance report. Outdated definitions leave the system vulnerable to new threats. |
| Event log and diagnostics | The OS and hardware keep logs (e.g. Windows Event Viewer, SMART for disks) that record errors and warnings. Review these during scheduled maintenance to identify recurring faults, plan preventive actions, and document corrective maintenance. Monthly review catches failing components before complete failure. |
| Backup and restore tools | Software for creating and verifying backups of user data and system state before corrective maintenance. Windows Backup, macOS Time Machine, or third-party tools create restore points and full backups. After corrective maintenance that involves OS reinstallation or disk replacement, backups are used to restore user files and settings. Verify backup integrity before relying on it for recovery. |

Annual Preventive Maintenance Schedule

| Maintenance Report | Description |
| --- | --- |
| Date and asset | Date of maintenance; asset ID or serial number; location. Recording the date and asset identifier enables tracking of maintenance history per asset over time. This information also supports warranty claims by providing evidence of when and where maintenance was performed. |
| Type of maintenance | Preventive (scheduled) or corrective; reference to schedule or job order if applicable. Distinguishing preventive from corrective maintenance helps management assess the effectiveness of the maintenance schedule. Over time, a high ratio of corrective to preventive work may indicate that the schedule needs to be revised or that equipment is nearing end of life. |
| Tasks performed | List of tasks (cleaning, parts replaced, updates applied, tests run). A detailed task listing enables trend analysis across multiple maintenance cycles and supports planning of future preventive schedules. Consistent documentation also ensures that another technician can review what was done and continue follow-up work if needed. |
| Parts and materials | Part numbers and quantities used; none if preventive only. Recording parts supports inventory management by tracking stock consumption and triggering reorder when levels are low. This data also assists with warranty claims and enables cost tracking per asset for budgeting purposes. |
| Sign-off | Technician name and date; user or supervisor acceptance if required. Sign-off provides accountability by confirming who performed the work and when it was completed. It also confirms that the system was tested and returned to the user in working condition. |

| Common Fault | Cause | Action |
| --- | --- | --- |
| Overheating / thermal shutdown | Dust block, failed fan, dried thermal paste | Clean vents and heatsink; replace fan; reapply thermal paste |
| No POST / no display | Loose RAM, faulty RAM, loose GPU | Reseat RAM and GPU; test with one stick; replace if faulty |
| Random crash / BSOD | Faulty RAM, driver issue, disk error | Run memory test; update or roll back driver; check disk SMART and event log |
| No boot / corrupted OS | Failed disk, bad update, malware | Repair or reinstall OS; replace disk if SMART failed; restore from backup if available |
| Noisy fan | Worn bearing, dust accumulation | Clean fan and surrounding area; replace fan if noise persists |
| USB / device not working | Loose cable, missing driver, port fault | Reseat cable; reinstall driver; try another port |

Learning Outcome Matrix

| Work Activity | Knowledge Outcome | Performance Outcome | Assessment Method | Evidence Required |
| --- | --- | --- | --- | --- |
| Identify computer maintenance requirements | Understand concepts related to: Identify computer maintenance requirements | Successfully execute: Identify computer maintenance requirements | Practical demonstration and documentation review | Completed task documentation and supervisor sign-off |
| Carry out computer scheduled preventive maintenance | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Perform computer corrective maintenance | Understand concepts related to: Perform computer corrective maintenance | Successfully execute: Perform computer corrective maintenance | Practical demonstration and documentation review | Completed task documentation and supervisor sign-off |
| Prepare computer maintenance report | Know tools, parts, and materials needed | Gather and organize tools, parts, and materials | Practical observation of tool/material preparation; checklist verification | Photo/log of prepared toolkit; materials checklist |

Practical Exercises

Lab 2.1: Identifying Maintenance Needs

Equipment Required

Test computer system, System monitoring software (HWiNFO, Event Viewer), BIOS/UEFI access, Temperature monitoring tools

Procedures

- 1. Boot the test system and log in

- 2. Open system monitoring software and check CPU, GPU, and disk temperatures

- 3. Open Windows Event Viewer and review System and Application logs for warnings or errors

- 4. Check SMART data on storage drives for early warnings

- 5. Inspect the system physically for dust buildup in vents and heatsinks

- 6. Document findings in a maintenance requirement checklist

- 7. Prioritize maintenance tasks based on urgency and risk

Expected Outcome

Completed maintenance requirement checklist with identified issues and priority ranking

Assessment Checklist

- System logs reviewed for errors or warnings

- Temperatures checked and documented

- SMART health status checked

- Physical inspection completed

- Maintenance needs documented

- Priority assigned to each task

Lab 2.2: Performing Preventive Maintenance

Equipment Required

Test computer, Compressed air, Soft anti-static brush, Lint-free cloth, Isopropyl alcohol, Thermal paste

Procedures

- 1. Power off the computer and unplug the power cable

- 2. Remove the side panel and visually inspect for dust accumulation

- 3. Use compressed air in short bursts to blow out dust from vents, fans, and heatsink

- 4. Use a soft brush to gently loosen dust from fan blades and tight spaces

- 5. Wipe down the motherboard and components with a lint-free cloth (no moisture)

- 6. Power on the system and verify all fans spin correctly and smoothly

- 7. Check temperatures in BIOS/UEFI to ensure cooling is working properly

- 8. Apply Windows/Linux updates if scheduled

- 9. Verify backup completion if automated backups are in place

Expected Outcome

Clean system with verified cooling function and current updates applied

Assessment Checklist

- System powered down and unplugged

- Dust removed from vents, fans, and heatsink

- Fans spinning correctly after restart

- Temperatures within normal range

- Updates applied successfully

- Backup status verified

Lab 2.3: Corrective Maintenance Response

Equipment Required

Test computer with simulated fault, Multimeter, Screwdrivers, Spare parts (RAM, fan, thermal paste), USB diagnostics media

Procedures

- 1. Receive a fault description (e.g., 'system overheating', 'blue screen')

- 2. Use diagnostics software to narrow down the issue

- 3. Perform physical inspection if hardware is suspected

- 4. For thermal issues: check heatsink seating, clean dust, reapply thermal paste

- 5. For memory errors: reseat RAM modules, test with single module

- 6. For fan failure: check BIOS for fan readings, replace fan if failed

- 7. Run stress tests to verify the issue is resolved

- 8. Document the fault, action taken, and parts replaced

Expected Outcome

System fully functional with fault corrected and documented

Assessment Checklist

- Fault correctly diagnosed

- Appropriate corrective action taken

- System stable after repair

- Replaced parts documented with serial numbers

- Testing confirms fault resolution

Lab 2.4: Maintenance Documentation

Equipment Required

Computer system, Maintenance report template, Pen and paper or text editor

Procedures

- 1. Gather all information: system asset ID, date, technician name

- 2. Summarize preventive or corrective tasks performed

- 3. Record parts replaced (if any) with serial numbers and costs

- 4. Document temperatures before and after maintenance

- 5. List any outstanding issues requiring future action

- 6. Include sign-off from technician and verification of completion

- 7. Attach or reference photos if major work was performed

Expected Outcome

Completed maintenance report with all required details

Assessment Checklist

- System asset ID recorded

- Date and technician documented

- All tasks listed with outcomes

- Parts replaced documented

- Next scheduled maintenance date recorded

- Report signed and dated

Employability Skills

This CoCu develops the following employability skills through the work activities:

- Activity: Prepare computer maintenance report

- Mapping: Document maintenance work clearly; record tasks performed, parts used, and sign-off; communicate findings to supervisor and users

- Activities: Identify computer maintenance requirements, Carry out preventive maintenance

- Mapping: Coordinate with help desk on maintenance schedules; share knowledge about trends and common issues; work with users to schedule downtime

- Activities: Perform computer corrective maintenance, Identify maintenance requirements

- Mapping: Diagnose root causes of hardware failures; determine whether to repair or replace components; troubleshoot using temperature software and diagnostics

- All maintenance activities

- Mapping: Take responsibility for systematic inspection; identify issues before user reports them; make decisions on appropriate preventive actions

- Activities: Identify computer maintenance requirements, Carry out scheduled preventive maintenance

- Mapping: Plan maintenance schedules; organize tools and replacement parts; follow preventive maintenance checklist in logical sequence

- All activities involving electrical components and thermal handling

- Mapping: Follow ESD precautions when handling RAM and boards; manage time to complete maintenance within scheduled windows; work safely with compressed air and thermal compounds

- All activities using diagnostic tools and hardware maintenance

- Mapping: Use temperature monitoring software, SMART diagnostics, and BIOS utilities; apply thermal paste correctly; use appropriate tools (screwdrivers, cable testers); perform firmware updates safely

- All activities; particularly for new hardware and failure modes

- Mapping: Stay informed on hardware failures and trends; learn from common faults documented in reports; adapt maintenance procedures based on field experience

Attitude, Safety and Environmental

- Electrostatic Discharge (ESD) Protection: Always wear an anti-static wrist strap and work on a grounded surface when handling motherboards, RAM, CPU, and expansion cards. Even small static charges can permanently damage sensitive components.

- Electrical Safety: When testing with a multimeter or working near power supplies, ensure the power is disconnected and use proper testing techniques. Follow all electrical safety guidelines and never work on live circuits.

- Thermal Compound Handling: Apply thermal paste correctly between CPU and heatsink to prevent overheating and damage. Use only the recommended amount and allow proper curing time before powering on.

- Safe Cable Management: Ensure network and power cables are properly routed to avoid tripping hazards, kinks, and damage. Label cables clearly for future troubleshooting and maintenance.

- E-waste Management: Properly dispose of defective hardware components, circuit boards, and packaging materials through authorised e-waste recycling centres. Never dispose of electronics in regular trash or landfill.

- Energy Efficiency: Configure systems to use power-saving modes and efficient power supplies. Avoid unnecessary idle time and follow organisational energy conservation policies.

- Punctuality and Reliability: Complete system maintenance work on schedule to meet user handover deadlines and support business operations.

- Integrity in Documentation: Prepare accurate and complete maintenance reports with all hardware details, software versions, and test results. Honest documentation supports future troubleshooting and maintains system integrity.

- Teamwork and Support: Collaborate with help desk staff and other technicians; share knowledge about system configurations and troubleshooting techniques to improve overall technical capability.

- Attention to Detail: Verify every connection, configuration setting, and test result to ensure maintenance tasks are completed correctly and systems function as intended.

References

# CoCu 3: Computer System Repair (180 hrs)

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION |  |
| --- | --- | --- |
| LEVEL | L3 |  |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 3: Computer System Repair |  |
| NO. AND WORK ACTIVITY STATEMENT | ASSESS COMPUTER REPAIR JOB ORDER/CHANGE REQUEST · CARRY OUT ONLINE TROUBLESHOOTING · PERFORM ON-SITE REPAIR · PREPARE COMPUTER STATUS REPORT |  |
| NO. CODE | IT-020-3:2013 - CoCu 3 / P(3/7) |  |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 3: Computer System Repair (Pembaikan Sistem Komputer) | CoCu 3: Computer System Repair (Pembaikan Sistem Komputer) |
| NO. AND WORK ACTIVITY STATEMENT | ASSESS COMPUTER REPAIR JOB ORDER/CHANGE REQUEST · CARRY OUT ONLINE TROUBLESHOOTING · PERFORM ON-SITE REPAIR · PREPARE COMPUTER STATUS REPORT | ASSESS COMPUTER REPAIR JOB ORDER/CHANGE REQUEST · CARRY OUT ONLINE TROUBLESHOOTING · PERFORM ON-SITE REPAIR · PREPARE COMPUTER STATUS REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 3 / P(3/7) | PAGE: 53 - 64 |

| SET-UP CONTEXT | ONLINE TROUBLESHOOTING | ON-SITE REPAIR |
| --- | --- | --- |
| Job request | Fault report from user describing symptoms (error message, no boot, slow performance). Initial assessment determines whether the fault can be resolved remotely. Job order includes asset ID, user contact, fault description, and priority. | Escalated from online troubleshooting when remote steps have not resolved the fault, or when hardware failure is suspected. Job order includes online troubleshooting history, fault description, and parts or tools likely needed. |
| Tools and parts | Phone or communication tool, remote desktop or remote support software, knowledge base, diagnostic scripts, and event log access. No physical parts needed. | Full toolkit: screwdrivers, multimeter, anti-static strap, spare parts (RAM, PSU, disk, cables), USB bootable media, and diagnostic software. Parts drawn from inventory based on the diagnosed fault. |
| Report | Record fault description, steps taken remotely, diagnostic findings, and outcome (resolved, pending, or escalated to on-site). | Computer status report with fault description, on-site actions taken, parts used (with serial numbers), test result, and recommendation (follow-up, disposal, or user training). |

| Type of Tools | Description |
| --- | --- |
| Phone / communication tool | Used to gather symptoms from the user, guide them step-by-step (e.g. restart, safe mode, check cables), and confirm the outcome. Clear questions and simple instructions reduce repeat visits. Document what the user reports and what was tried for the status report. |
| Remote desktop / remote support tool | Used when the user can boot and connect to the network: view their screen, run diagnostics (event log, disk check, memory test), remove malware or bad drivers, and apply fixes without visiting. Requires user permission and network access; document all steps for the status report. |
| Knowledge base / diagnostic scripts | Used to look up error codes, beep codes, and known fixes; run scripted checks (e.g. disk SMART, memory test) to narrow the fault. Speeds up both online and on-site repair and ensures consistent procedure across technicians. |
| Screwdriver set and anti-static strap | Used to open the chassis, replace RAM, GPU, PSU, or drives during on-site repair. Anti-static strap is worn and connected to a grounded point when handling boards to avoid ESD damage. Essential for any hardware replacement. |
| Multimeter | Used to check PSU voltages (+3.3 V, +5 V, +12 V), continuity, and power at the connector. Confirms PSU fault before replacement and checks front-panel wiring. Helps isolate electrical faults that are not visible. |
| Spare parts and USB installer | Known-good RAM, PSU, or disk for swap testing; USB bootable media for OS reinstall or recovery. Verify compatibility (socket, form factor, interface) before use; record serial numbers for the status report. Swap testing isolates the faulty component by replacing one part at a time. |
| Diagnostic software | Memory test (e.g. Memtest86), disk SMART tools, event log viewer, and temperature monitors. Run after repair to verify stability and document the test result in the computer status report. |

| % | Hrs | Work activity | Knowledge 30% | Performance 70% | Total |
| --- | --- | --- | --- | --- | --- |
| 10% | 18 | Assess computer repair job order/change request | 5.4 | 12.6 | 18.0 |
| 30% | 54 | Carry out online trouble shooting | 16.2 | 37.8 | 54.0 |
| 50% | 90 | Perform on-site repair | 27.0 | 63.0 | 90.0 |
| 10% | 18 | Prepare computer status report | 5.4 | 12.6 | 18.0 |
| 100% | 180 |  | 54.0 | 126.0 | 180.0 |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 3: Computer System Repair (Pembaikan Sistem Komputer) | CoCu 3: Computer System Repair (Pembaikan Sistem Komputer) |
| NO. AND WORK ACTIVITY STATEMENT | ASSESS COMPUTER REPAIR JOB ORDER/CHANGE REQUEST · CARRY OUT ONLINE TROUBLESHOOTING · PERFORM ON-SITE REPAIR · PREPARE COMPUTER STATUS REPORT | ASSESS COMPUTER REPAIR JOB ORDER/CHANGE REQUEST · CARRY OUT ONLINE TROUBLESHOOTING · PERFORM ON-SITE REPAIR · PREPARE COMPUTER STATUS REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 3 / P(3/7) | PAGE: 65 - 72 |

Computer System Troubleshooting Flowchart

| Hardware Component | Description |
| --- | --- |
| POST (Power-On Self-Test) | Firmware checks run at boot that test RAM, drives, keyboard, and other devices. Beep codes or on-screen codes indicate which check failed. Consult the motherboard or system manual for the meaning of codes. No beep or no display often points to PSU, motherboard, RAM, or CPU failure. Understanding POST is essential for on-site diagnosis before opening the chassis. |
| Power supply unit (PSU) | Converts AC to DC and supplies voltages to the motherboard and components. A failed or degrading PSU may cause no power, random shutdowns, or instability. Use a multimeter to check +3.3 V, +5 V, and +12 V rails at the connector. Replace with a unit of equal or greater wattage and matching connector types. Record the replacement part serial number in the status report. |
| RAM modules | Temporary storage for running programs. Faulty RAM causes no POST, random crashes, or blue screens. Run Memtest86 or Windows Memory Diagnostic to confirm. During on-site repair, reseat modules first; if the fault persists, test each stick individually in a known-good slot. Replace faulty modules with compatible type (DDR5) and speed. |
| Storage drives (HDD / SSD / NVMe) | Failed or failing drives cause no boot, data loss, or slow performance. Check SMART data for warnings (reallocated sectors, pending sectors). Back up data if possible before replacement. Replace with a compatible drive and reinstall the OS or restore from backup. Record the old and new drive serial numbers. |

| Software Component | Description |
| --- | --- |
| Safe mode / recovery environment | The OS loads with minimal drivers and services. Used to isolate software versus hardware faults and to remove malware or bad drivers. Recovery options (e.g. Windows Recovery Environment) allow restore point, system repair, or full reinstall. Accessible via boot menu or installation media. |
| Event log and minidump analysis | Windows Event Viewer (Application and System logs) and minidump files record errors, warnings, and crash details. Review these during online troubleshooting to identify the failing component or driver. Repeated errors for the same source suggest a persistent fault that requires corrective action. |
| Replacement parts compatibility | Use parts that match the system's specifications: same socket, form factor, interface, and supported speed. Check the motherboard or system manual before ordering. Record serial numbers and warranty information for the status report. Verify compatibility before swap testing to avoid introducing new faults. |
| Backup verification | Before performing destructive repairs (OS reinstall, disk replacement), verify that a current backup exists and is restorable. Backup tools include Windows Backup, cloud sync (OneDrive, Google Drive), and external media. Confirming backup status protects the user's data and reduces the risk of data loss during repair. |

| Computer Status Report | Description |
| --- | --- |
| Fault description | Clear statement of the reported fault and observed symptoms during diagnosis. Accurate fault descriptions allow future technicians to recognise recurring patterns and speed up diagnosis of similar issues. This record also provides evidence for management when deciding whether to repair or replace an aging asset. |
| Actions taken | All steps performed during online troubleshooting and on-site repair, in sequence. Documenting each step in order prevents repeated work if another technician takes over and provides a reference for similar faults in the future. This also demonstrates due diligence and compliance with the organisation's repair procedures. |
| Parts used | Part numbers, serial numbers, and quantities of replacement components. Recording serial numbers ties each part to a specific asset for warranty tracking and audit purposes. This information also supports inventory management by updating stock records and enabling cost analysis per repair. |
| Test result | Outcome of post-repair testing (POST, boot, diagnostics, stress test if required). Documenting test results provides evidence that the system was verified as functional before being returned to the user. If the fault recurs, the previous test results help narrow down whether the original repair was effective or a different component is now failing. |
| Recommendation | Follow-up actions such as monitoring, user training, further repair, or disposal. Recommendations guide management in planning next steps, such as scheduling a follow-up check or budgeting for a replacement. Including user training suggestions helps reduce repeat fault reports caused by operator error. |

| Common Fault | Cause | Action |
| --- | --- | --- |
| No power / no POST | PSU fault, loose power cable, wrong front-panel wiring, or motherboard failure | Check cable and switch; verify front-panel header; test PSU with multimeter; replace or escalate |
| No display | Loose or wrong cable, wrong monitor input, GPU or RAM not seated, or faulty GPU | Check cable and input; reseat GPU and RAM; try onboard video if available |
| Blue screen / crash | Driver conflict, faulty RAM, or disk error; overheating | Check event log and minidump; run memory and disk test; check temperatures |
| Slow or freezing | Malware, too many startup programs, low RAM, or failing disk | Scan for malware; check startup programs and RAM usage; check SMART and disk health |
| No boot / corrupted OS | Failed disk, bad OS update, or malware damage | Attempt repair via recovery environment; reinstall OS; replace disk if SMART shows failure; restore from backup |

Learning Outcome Matrix

| Work Activity | Knowledge Outcome | Performance Outcome | Assessment Method | Evidence Required |
| --- | --- | --- | --- | --- |
| Assess computer repair job order/change request | Understand concepts related to: Assess computer repair job order/change request | Successfully execute: Assess computer repair job order/change request | Practical demonstration and documentation review | Completed task documentation and supervisor sign-off |
| Carry out online trouble shooting | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Perform on-site repair | Understand concepts related to: Perform on-site repair | Successfully execute: Perform on-site repair | Practical demonstration and documentation review | Completed task documentation and supervisor sign-off |
| Prepare computer status report | Know tools, parts, and materials needed | Gather and organize tools, parts, and materials | Practical observation of tool/material preparation; checklist verification | Photo/log of prepared toolkit; materials checklist |

Practical Exercises

Lab 3.1: Fault Analysis and Diagnosis

Equipment Required

Faulty computer system, Diagnostics software, Hardware reference guides, Multimeter

Procedures

- 1. Receive a fault report describing symptoms (no power, slow performance, error messages)

- 2. Ask clarifying questions: when did it start, what was the last action before failure

- 3. Run diagnostics software to identify failing components

- 4. Check event logs for clues about the fault cause

- 5. Use a multimeter to check power supply voltages if power-related

- 6. Develop a repair strategy addressing the root cause

- 7. Document the analysis and proposed repair steps

Expected Outcome

Fault analysis report with diagnosed problem and proposed repair plan

Assessment Checklist

- Symptoms documented

- Diagnostics run successfully

- Logs reviewed for errors

- Root cause identified

- Repair strategy documented

- Required parts/tools listed

Lab 3.3: Remote Troubleshooting and Online Support

Equipment Required

Customer computer (remote access), Remote support software, Knowledge base or diagnostic scripts, Phone or chat for communication

Procedures

- 1. Contact the user and gather detailed symptom information (error messages, when it started, what was the last action)

- 2. Ask the user to check basic connectivity (network, power, cables)

- 3. Request remote desktop access or use remote support software to view the user's screen

- 4. Run diagnostics remotely: open Event Viewer, check Device Manager, run memory test if available

- 5. Guide the user through basic troubleshooting steps: restart, Safe Mode boot, check cables

- 6. Look up error codes in knowledge base and apply known fixes

- 7. If remotely fixable, apply the fix and confirm resolution with the user

- 8. If hardware fault is likely, escalate to on-site repair and document findings

Expected Outcome

Fault diagnosed; issue resolved remotely or escalated with documentation for on-site repair

Assessment Checklist

- Symptom information fully documented

- Remote access established

- Diagnostics run and logs reviewed

- Troubleshooting steps documented

- Issue resolved OR escalated with clear handoff

- User confirmation obtained

Lab 3.2: Hardware Component Replacement and On-site Repair

Equipment Required

Computer with faulty component, Replacement component, Screwdrivers, Anti-static strap, Thermal paste (if CPU/heatsink)

Procedures

- 1. Power off and unplug the computer

- 2. Ground yourself with an anti-static strap

- 3. Identify the faulty component location in the system

- 4. Remove any screws or clips holding the component

- 5. Carefully extract the faulty component

- 6. Inspect the replacement component for any visible damage

- 7. Install the replacement component securely

- 8. Reapply thermal paste if replacing CPU or GPU heatsink

- 9. Power on and verify the system boots correctly

- 10. Run diagnostics to confirm the fault is resolved

Expected Outcome

System repaired with faulty component replaced and verified working

Assessment Checklist

- Faulty component identified and removed safely

- Replacement component installed correctly

- System boots without errors

- Diagnostics pass

- All cables reconnected properly

- Repair verified complete

Lab 3.4: Computer Status Report and Repair Documentation

Equipment Required

Repaired computer, Status report template, Pen and paper or text editor, Camera (optional for photos)

Procedures

- 1. Record the original fault description and symptoms from the job order

- 2. Document online troubleshooting steps performed and findings (if applicable)

- 3. Record on-site repair actions taken and components accessed or replaced

- 4. List replaced hardware with part numbers and serial numbers for inventory tracking

- 5. Record technician time spent on diagnosis and repair

- 6. Include test results (temperature readings, diagnostics) confirming repair success

- 7. Note any warranty, service tags, or asset ID labels applied

- 8. Add recommendation: follow-up maintenance, user training, or hardware upgrade suggestions

- 9. Obtain user or supervisor sign-off if required by policy

Expected Outcome

Complete status report documenting initial fault, troubleshooting, repair actions, parts used, test results, and recommendations

Assessment Checklist

- Fault description clear and complete

- Troubleshooting steps documented

- On-site repair actions detailed

- Parts replaced with serial numbers recorded

- Test results showing success

- Recommendations included

- Report signed and dated

Employability Skills

This CoCu develops the following employability skills through the work activities:

- Activity: Prepare computer status report

- Mapping: Document repair work clearly; explain faults and solutions to users and supervisors; prepare status reports with findings and recommendations

- Activities: Assess computer repair job order, Carry out online troubleshooting

- Mapping: Work with help desk and support staff; escalate complex issues appropriately; ask technical experts for advice on difficult repairs

- Activities: Carry out online troubleshooting, Perform on-site repair

- Mapping: Diagnose root causes using systematic troubleshooting; isolate hardware vs software issues; use event logs, diagnostics, and multimeters to identify faults

- Activities: Assess computer repair job order, Perform on-site repair

- Mapping: Take responsibility for repair decisions; work independently to resolve on-site issues; manage user expectations and communicate timelines

- Activities: Assess computer repair job order, Carry out troubleshooting

- Mapping: Plan troubleshooting sequence logically; organize tools and spare parts for on-site visits; prioritize high-impact repairs

- All repair activities, especially those involving electrical components

- Mapping: Work safely with multimeters and power supplies; manage time to complete repairs efficiently; maintain composure when troubleshooting complex issues; follow safety protocols for on-site work

- All activities requiring diagnostic and repair skills

- Mapping: Use multimeters, diagnostic software, and event viewers; interpret SMART data and memory test results; perform hands-on repairs with precision

- All activities; especially relevant for new hardware and error patterns

- Mapping: Learn new troubleshooting techniques; stay informed on common hardware failures and malware signatures; analyze repair cases to improve future efficiency

Attitude, Safety and Environmental

- Electrostatic Discharge (ESD) Protection: Always wear an anti-static wrist strap and work on a grounded surface when handling motherboards, RAM, CPU, and expansion cards. Even small static charges can permanently damage sensitive components.

- Electrical Safety: When testing with a multimeter or working near power supplies, ensure the power is disconnected and use proper testing techniques. Follow all electrical safety guidelines and never work on live circuits.

- Thermal Compound Handling: Apply thermal paste correctly between CPU and heatsink to prevent overheating and damage. Use only the recommended amount and allow proper curing time before powering on.

- Safe Cable Management: Ensure network and power cables are properly routed to avoid tripping hazards, kinks, and damage. Label cables clearly for future troubleshooting and maintenance.

- E-waste Management: Properly dispose of defective hardware components, circuit boards, and packaging materials through authorised e-waste recycling centres. Never dispose of electronics in regular trash or landfill.

- Energy Efficiency: Configure systems to use power-saving modes and efficient power supplies. Avoid unnecessary idle time and follow organisational energy conservation policies.

- Punctuality and Reliability: Complete system repair work on schedule to meet user handover deadlines and support business operations.

- Integrity in Documentation: Prepare accurate and complete repair status reports with all hardware details, software versions, and test results. Honest documentation supports future troubleshooting and maintains system integrity.

- Teamwork and Support: Collaborate with help desk staff and other technicians; share knowledge about system configurations and troubleshooting techniques to improve overall technical capability.

- Attention to Detail: Verify every connection, configuration setting, and test result to ensure repair tasks are completed correctly and systems function as intended.

References

# CoCu 4: Server Installation (240 hrs)

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION |  |
| --- | --- | --- |
| LEVEL | L3 |  |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 4: Server Installation |  |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB ORDER/CHANGE REQUEST · EXECUTE HARDWARE INSTALLATION · CARRY OUT SOFTWARE INSTALLATION · PERFORM SERVER FUNCTIONALITY TEST · PREPARE SERVER INSTALLATION SET-UP REPORT |  |
| NO. CODE | IT-020-3:2013 - CoCu 4 / P(4/7) |  |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 4: Server Installation (Pemasangan Pelayan) | CoCu 4: Server Installation (Pemasangan Pelayan) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB ORDER/CHANGE REQUEST · EXECUTE HARDWARE INSTALLATION · CARRY OUT SOFTWARE INSTALLATION · PERFORM SERVER FUNCTIONALITY TEST · PREPARE SERVER INSTALLATION SET-UP REPORT | ANALYSE JOB ORDER/CHANGE REQUEST · EXECUTE HARDWARE INSTALLATION · CARRY OUT SOFTWARE INSTALLATION · PERFORM SERVER FUNCTIONALITY TEST · PREPARE SERVER INSTALLATION SET-UP REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 4 / P(4/7) | PAGE: 73 - 92 |

| SET-UP CONTEXT | TOWER SERVER | RACK SERVER |
| --- | --- | --- |
| Job request | Small business or branch office requiring a standalone server. Job order specifies server model, OS, roles (e.g. file server, DNS), and network settings. Single PSU and standard cooling; placed on floor or in cabinet. | Data centre or server room deployment requiring rack-mounted servers. Job order specifies rack position, form factor (1U, 2U), redundant PSU, RAID configuration, network VLAN, and out-of-band management (iLO, iDRAC). |
| Tools and parts | Standard server toolkit: screwdrivers, anti-static strap, USB installer, and network cables. Server-grade components (CPU, RAM, drives, RAID card) from inventory. No rack hardware needed. | Extended toolkit: rack rails, cage nuts, cable management arms, label maker, screwdrivers, anti-static strap, USB installer, and network cables. Hot-swap drives and redundant PSUs as specified. PDU connections and cable labelling required. |
| Report | Server installation report with asset ID, location, hardware configuration, OS and roles, network settings (IP, gateway, DNS), and sign-off. | Server installation report with asset ID, rack position, hardware configuration (including RAID), redundant PSU status, management IP, OS and roles, network settings (IP, VLAN, gateway, DNS), and sign-off. |

| Type of Tools | Description |
| --- | --- |
| Screwdriver set and anti-static strap | Used to install server components (CPU, RAM, drives, expansion cards) and to mount the server in a rack or cabinet. Anti-static strap is worn and connected to a grounded point when handling server boards and memory to prevent ESD damage. Server components are more costly than desktop parts, making ESD precautions critical. |
| Rack rails and cage nuts | Used to mount rack servers into the server rack. Rails are specific to the server model and rack type; verify compatibility before installation. Cage nuts are inserted into the rack posts to provide threaded mounting points. Correct rail installation ensures the server slides in and out smoothly for maintenance. |
| Cable management arms and label maker | Cable management arms attach to the rear of the server and organise power, network, and management cables so the server can slide out without disconnecting. A label maker is used to tag each cable and port for identification during troubleshooting and maintenance. Proper labelling reduces errors in multi-server environments. |
| Multimeter | Used to check power supply voltages and continuity before and after installation. Confirms that the PSU is within tolerance and that PDU outlets are providing correct voltage. Essential when commissioning redundant PSU configurations to verify both power paths are active. |
| USB bootable media | Used to load the server operating system (Windows Server, Linux) and drivers during software installation. The installer is prepared according to the job order with the correct OS version and licence key. Some servers require legacy boot or UEFI settings to be configured in BIOS before booting from USB. |
| Network cable tester | Used to verify network cable wiring and connectivity between the server NIC and the switch port. Confirms that each wire is correctly terminated and that the link is active at the expected speed. Essential when connecting multiple NICs or configuring VLAN trunks. |

| % | Hrs | Work activity | Knowledge 30% | Performance 70% | Total |
| --- | --- | --- | --- | --- | --- |
| 15% | 36 | Analyse job order/change request | 10.8 | 25.2 | 36.0 |
| 40% | 96 | Execute hardware installation | 28.8 | 67.2 | 96.0 |
| 30% | 72 | Carry out software installation | 21.6 | 50.4 | 72.0 |
| 10% | 24 | Perform server functionality test | 7.2 | 16.8 | 24.0 |
| 5% | 12 | Prepare server installation set-up report | 3.6 | 8.4 | 12.0 |
| 100% | 240 |  | 72.0 | 168.0 | 240.0 |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 4: Server Installation (Pemasangan Pelayan) | CoCu 4: Server Installation (Pemasangan Pelayan) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB ORDER/CHANGE REQUEST · EXECUTE HARDWARE INSTALLATION · CARRY OUT SOFTWARE INSTALLATION · PERFORM SERVER FUNCTIONALITY TEST · PREPARE SERVER INSTALLATION SET-UP REPORT | ANALYSE JOB ORDER/CHANGE REQUEST · EXECUTE HARDWARE INSTALLATION · CARRY OUT SOFTWARE INSTALLATION · PERFORM SERVER FUNCTIONALITY TEST · PREPARE SERVER INSTALLATION SET-UP REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 4 / P(4/7) | PAGE: 93 - 112 |

| Hardware Component | Description |
| --- | --- |
| Server chassis / form factor | Rack servers (1U, 2U, etc.) are mounted in a standard 19-inch rack with rails and require proper cooling and power distribution. Tower servers are standalone chassis similar to desktops, placed on the floor or in a cabinet, and are easier to access for small sites. The choice is determined by the job order based on site requirements, number of servers, and available infrastructure. |
| Redundant power supply (PSU) | Dual or hot-swap PSUs provide continuous power if one unit fails. Both PSUs are connected to different circuits or power distribution units (PDUs) where specified so that a single power failure does not shut down the server. Verify both PSUs are active and drawing load after installation using the server management interface or front-panel indicators. |
| RAID controller and disk array | Redundant array of independent disks for capacity and/or data redundancy (e.g. RAID 1 for mirroring, RAID 5 for parity, RAID 10 for mirroring plus striping). Configured in BIOS/UEFI or via a hardware RAID card before installing the OS. The operating system is installed after the array is created and recognised by the installer. Hot-swap drive bays allow failed disks to be replaced without shutting down the server. |
| Network interfaces (NICs) | One or more network interface cards providing connectivity to the LAN, management network, or storage network. IP address, subnet, gateway, and VLAN are configured as per the job order. Cables are connected to the correct switch ports and labelled. Teaming or bonding may be configured for redundancy or bandwidth. |
| Out-of-band management (iLO / iDRAC) | Remote management interfaces such as HP iLO or Dell iDRAC that provide console access, power control, and hardware monitoring even when the OS is down. A dedicated management IP and credentials are configured per the job order. Enables remote troubleshooting and reduces the need for physical access to the server room. |

Standard 42U Server Rack Layout

RAID Level Comparison

| Software Component | Description |
| --- | --- |
| Server operating system | Windows Server or Linux (e.g. CentOS, Ubuntu Server, RHEL) installed from USB or network boot as per the job order. Installation includes partitioning, base OS configuration, hostname, and initial administrator credentials. The licence or activation key is applied during or after installation. |
| Roles and features | Server roles such as file server, DNS, DHCP, Active Directory, Hyper-V, or web server are installed and configured as specified in the change request. Dependencies must be followed (e.g. Active Directory before file server permissions). Each role is tested after installation to confirm correct operation. |
| Patches and updates | Security and cumulative updates applied after the base OS installation to bring the server to current patch level. Reboots are scheduled as required; downtime is communicated to stakeholders. Verify that updates install correctly and that all roles function normally after reboot. |
| Backup and recovery configuration | Backup software and schedule configured as part of the server installation to protect data from the first day of operation. Includes configuring Windows Server Backup or a third-party agent (e.g. Veeam, Acronis), defining backup targets (local disk, NAS, or cloud), and verifying the first backup completes successfully. The backup configuration is documented in the server installation report. |

| Server Installation Report | Description |
| --- | --- |
| Asset and location | Server name, asset ID, rack position (for rack servers), or physical location (for tower servers). Recording the exact location enables quick identification during maintenance or incident response, especially in multi-rack environments. This information also feeds into the organisation's asset register for lifecycle tracking and capacity planning. |
| Hardware configuration | Model, CPU, RAM capacity, storage configuration (RAID level, number and type of drives), NICs, and management IP. A complete hardware record ensures that future upgrades or replacements use compatible components and that capacity can be assessed without physical inspection. This also serves as the baseline for troubleshooting, allowing technicians to compare the current state against the original build specification. |
| Software configuration | OS version, installed roles and features, key applications, and patch level. Documenting the software baseline ensures that any changes made after deployment can be compared against the original installation. This record is essential for disaster recovery, as it provides the information needed to rebuild the server to its exact pre-failure state. |
| Network settings | IP addresses, subnet mask, VLAN, gateway, and DNS servers. Accurate network records prevent IP conflicts and enable rapid reconfiguration if the server is moved or the network is restructured. This information is also critical for firewall rule creation and network security audits. |
| Sign-off | Installer name and date; acceptance by requester or supervisor. Sign-off provides accountability by confirming who performed the installation and that the requester accepted the completed work. It also marks the formal handover point after which the server enters production and is covered by the organisation's maintenance and support processes. |

| Common Fault | Cause | Action |
| --- | --- | --- |
| Server does not POST | Loose CPU, RAM not seated, PSU not connected, or motherboard fault | Check all power connections; reseat CPU and RAM; verify PSU switch and cables; test with minimal configuration |
| RAID array not recognised | RAID controller not configured, drives not seated, or incompatible drives | Enter RAID BIOS and create array; reseat drives; verify drive compatibility with controller |
| OS installer does not detect RAID | Missing RAID driver for the OS installer | Load the RAID controller driver from USB during OS installation; download correct driver from manufacturer |
| No network connectivity after install | NIC driver missing, wrong IP configuration, or cable not connected to correct switch port | Install NIC driver; verify IP settings match job order; check cable and switch port; test with ping |
| Out-of-band management not accessible | Management IP not configured, wrong VLAN, or cable not connected | Configure management IP in BIOS/UEFI or via front-panel; verify cable is connected to the management network port; check VLAN settings |
| Redundant PSU not active | Second PSU not connected, PDU circuit off, or PSU fault | Connect second PSU to a separate PDU or circuit; verify power indicator on both PSUs; replace faulty PSU |

Learning Outcome Matrix

| Work Activity | Knowledge Outcome | Performance Outcome | Assessment Method | Evidence Required |
| --- | --- | --- | --- | --- |
| Analyse job order/change request | Identify and interpret requirements | Analyse requirements and extract key priorities | Written test on requirement analysis; oral questions on decision criteria | Completed analysis checklist; documented decision rationale |
| Execute hardware installation | Understand installation procedures and configuration options | Successfully install and configure software or systems | Practical installation; verification of correct configuration | Installation log; system configuration screenshots; test results |
| Carry out software installation | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Perform server functionality test | Know testing procedures and acceptance criteria | Execute tests and document results | Practical test execution; result documentation | Test results log; signed-off test report |
| Prepare server installation set-up report | Know tools, parts, and materials needed | Gather and organize tools, parts, and materials | Practical observation of tool/material preparation; checklist verification | Photo/log of prepared toolkit; materials checklist |

Practical Exercises

Lab 1: Analyse job order/change request

Procedures

- 1. Follow established procedures for analyse job order/change request

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of analyse job order/change request with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 2: Execute hardware installation

Procedures

- 1. Follow established procedures for execute hardware installation

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of execute hardware installation with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 3: Carry out software installation

Procedures

- 1. Follow established procedures for carry out software installation

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of carry out software installation with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 4: Perform server functionality test

Procedures

- 1. Follow established procedures for perform server functionality test

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of perform server functionality test with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 5: Prepare server installation set-up report

Procedures

- 1. Follow established procedures for prepare server installation set-up report

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of prepare server installation set-up report with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Employability Skills

This CoCu develops the following employability skills through the work activities:

- Activity: Analyse job order/change request

- Mapping: Clarify server requirements with stakeholders; document configuration details; communicate completion status to project team

- All activities

- Mapping: Coordinate with network teams on IP allocation and cable management; work with storage teams on RAID configuration; involve supervisor in complex decisions

- All activities, especially during functionality testing

- Mapping: Diagnose POST failures, RAID issues, and driver problems; troubleshoot network connectivity using systematic methods

- All server installation activities

- Mapping: Work independently on hardware assembly and configuration; take responsibility for verification testing before handover

- Activities: Analyse job order, Execute hardware installation

- Mapping: Plan installation sequence; organize tools and components; follow correct BIOS/firmware configuration order

- All activities involving high-power systems and complex configurations

- Mapping: Follow safety protocols for server power distribution; manage time to complete installation on schedule; maintain detailed documentation for production handover

- All activities requiring server hardware, RAID, network, and OS installation skills

- Mapping: Configure RAID arrays and network settings; perform BIOS/UEFI configuration; troubleshoot using diagnostic tools and management interfaces

- All activities; especially for different server platforms and configurations

- Mapping: Learn new server hardware architectures; stay current on RAID controller and firmware updates; analyze installation issues to improve future procedures

Attitude, Safety and Environmental

- Electrostatic Discharge (ESD) Protection: Always wear an anti-static wrist strap and work on a grounded surface when handling motherboards, RAM, CPU, and expansion cards. Even small static charges can permanently damage sensitive components.

- Electrical Safety: When testing with a multimeter or working near power supplies, ensure the power is disconnected and use proper testing techniques. Follow all electrical safety guidelines and never work on live circuits.

- Thermal Compound Handling: Apply thermal paste correctly between CPU and heatsink to prevent overheating and damage. Use only the recommended amount and allow proper curing time before powering on.

- Safe Cable Management: Ensure network and power cables are properly routed to avoid tripping hazards, kinks, and damage. Label cables clearly for future troubleshooting and maintenance.

- E-waste Management: Properly dispose of defective hardware components, circuit boards, and packaging materials through authorised e-waste recycling centres. Never dispose of electronics in regular trash or landfill.

- Energy Efficiency: Configure systems to use power-saving modes and efficient power supplies. Avoid unnecessary idle time and follow organisational energy conservation policies.

- Punctuality and Reliability: Complete server installation work on schedule to meet user handover deadlines and support business operations.

- Integrity in Documentation: Prepare accurate and complete installation reports with all hardware details, software versions, and test results. Honest documentation supports future troubleshooting and maintains system integrity.

- Teamwork and Support: Collaborate with help desk staff and other technicians; share knowledge about system configurations and troubleshooting techniques to improve overall technical capability.

- Attention to Detail: Verify every connection, configuration setting, and test result to ensure server installation tasks are completed correctly and systems function as intended.

References

# CoCu 5: Server Maintenance (180 hrs)

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION |  |
| --- | --- | --- |
| LEVEL | L3 |  |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 5: Server Maintenance |  |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE SERVER MAINTENANCE JOB ORDER · CARRY OUT HARDWARE MAINTENANCE · PERFORM SERVER OPERATING SYSTEM MAINTENANCE · PREPARE SERVER MAINTENANCE RECORD |  |
| NO. CODE | IT-020-3:2013 - CoCu 5 / P(5/7) |  |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 5: Server Maintenance (Penyelenggaraan Pelayan) | CoCu 5: Server Maintenance (Penyelenggaraan Pelayan) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE SERVER MAINTENANCE JOB ORDER · CARRY OUT HARDWARE MAINTENANCE · PERFORM SERVER OPERATING SYSTEM MAINTENANCE · PREPARE SERVER MAINTENANCE RECORD | ANALYSE SERVER MAINTENANCE JOB ORDER · CARRY OUT HARDWARE MAINTENANCE · PERFORM SERVER OPERATING SYSTEM MAINTENANCE · PREPARE SERVER MAINTENANCE RECORD |
| NO. CODE | IT-020-3:2013 - CoCu 5 / P(5/7) | PAGE: 113 - 122 |

| SET-UP CONTEXT | RACK SERVER / PREVENTIVE MAINTENANCE | TOWER SERVER / CORRECTIVE MAINTENANCE |
| --- | --- | --- |
| Job order | Scheduled preventive maintenance for a rack-mounted server in the server room. Job order specifies server TAG/ID, OS version, warranty status, and maintenance checklist (hardware inspection, backup verification, log review, patch application). | Corrective maintenance for a tower server after hardware fault alert. Change request specifies server TAG/ID, fault description (e.g. degraded RAID, failed PSU), replacement parts, and expected downtime window. |
| Tools and parts | Standard server toolkit: vacuum cleaner, cable ties, screwdriver set, anti-static strap. Backup media (external drive or tape). Monitoring software access credentials and server access pass. | Extended toolkit: multimeter, replacement components (hot-swap disk, PSU), anti-static mat, manufacturer operating manual. Cable tester for network verification after repair. RAID controller documentation for rebuild procedures. |
| Record | Preventive maintenance record with server utilisation status, LED indicator readings, backup verification, log review summary, and technician sign-off. | Corrective maintenance record with fault description, root cause, parts replaced, warranty claim reference, RAID rebuild status, downtime duration, and user sign-off. Includes server maintenance costing estimate. |

| Type of Tools | Description |
| --- | --- |
| Vacuum cleaner (anti-static / ESD-safe) | Used to remove dust from server vents, heatsinks, fans, and internal components. Dust build-up restricts airflow and causes overheating. Only ESD-safe vacuum cleaners should be used in server rooms to avoid electrostatic discharge damage to sensitive components. Clean servers on a scheduled basis as part of preventive maintenance. |
| Screwdriver set (Phillips, flat, Torx) | Used to open server chassis, remove drive caddies, secure expansion cards, and replace hot-swap components. Server hardware often uses Torx or proprietary screws. Keep the correct bit sizes for each server model to avoid stripping. |
| Cable ties and labels | Used to organise and label server cables (power, network, SAN, KVM) for identification and airflow management. Proper cable management prevents accidental disconnection, improves cooling, and simplifies troubleshooting. Labels should include port number, destination, and VLAN or network segment where applicable. |
| Multimeter | Used to verify PSU voltages (+3.3 V, +5 V, +12 V) and check for short circuits before and after maintenance. Ensures replacement power supplies are within tolerance. Essential for corrective maintenance when a power-related fault is suspected. |
| Anti-static mat and wrist strap | Prevents electrostatic discharge when handling server components (RAM, CPU, expansion cards, drives). The wrist strap is connected to a grounded point; the mat provides a grounded working surface. Required for all internal hardware work on servers. |
| Backup software and media | Used to perform and verify server data backups before maintenance. Backup media includes external storage drives, tape cartridges, or network-attached storage. Backup software manages full and incremental backups and generates completion logs for verification. |
| Server management / monitoring software | Remote management tools (e.g. iDRAC, iLO, IPMI) and monitoring dashboards used to check server utilisation (CPU, memory, disk, network), LED status indicators, system logs, and antivirus status. Essential for analysing maintenance requirements and verifying post-maintenance health. |

| % | Hrs | Work activity | Knowledge 30% | Performance 70% | Total |
| --- | --- | --- | --- | --- | --- |
| 15% | 27 | Analyse server maintenance job order | 8.1 | 18.9 | 27.0 |
| 45% | 81 | Carry out hardware maintenance | 24.3 | 56.7 | 81.0 |
| 30% | 54 | Perform server Operating System maintenance | 16.2 | 37.8 | 54.0 |
| 10% | 18 | Prepare server maintenance record | 5.4 | 12.6 | 18.0 |
| 100% | 180 |  | 54.0 | 126.0 | 180.0 |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 5: Server Maintenance (Penyelenggaraan Pelayan) | CoCu 5: Server Maintenance (Penyelenggaraan Pelayan) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE SERVER MAINTENANCE JOB ORDER · CARRY OUT HARDWARE MAINTENANCE · PERFORM SERVER OPERATING SYSTEM MAINTENANCE · PREPARE SERVER MAINTENANCE RECORD | ANALYSE SERVER MAINTENANCE JOB ORDER · CARRY OUT HARDWARE MAINTENANCE · PERFORM SERVER OPERATING SYSTEM MAINTENANCE · PREPARE SERVER MAINTENANCE RECORD |
| NO. CODE | IT-020-3:2013 - CoCu 5 / P(5/7) | PAGE: 123 - 132 |

| Hardware Component | Description |
| --- | --- |
| Server chassis and rack mount | The physical enclosure that houses all server components. Rack-mounted servers (1U, 2U, 4U) slide into standard 19-inch racks; tower servers stand alone. During maintenance, check the chassis for physical damage, dust accumulation, and proper airflow. Ensure rack rails and cable management arms are secure. The chassis must be accessible for hot-swap operations and cleaning. |
| Server power supply unit (PSU) | Provides power to all server components. Enterprise servers typically have redundant PSUs (e.g. 1+1) that allow hot-swap replacement without downtime. During maintenance, check PSU LED indicators: green means normal, amber or red indicates a fault. Use a multimeter to verify voltages are within tolerance. Replace faulty PSUs promptly to maintain redundancy. |
| Server fans and cooling system | Maintain operating temperature inside the server. Servers have multiple fans with redundancy; a failed fan triggers an alert on the management interface and an amber LED. During maintenance, check fan speed, listen for unusual noise, and clean dust from blades and heatsinks. Server room temperature (18–27 °C) and humidity (40–60% RH) must be within range. |
| Hard disk drives (HDD / SSD) in RAID array | Server storage configured in RAID (Redundant Array of Independent Disks) for performance and data protection. RAID levels include RAID 1 (mirror), RAID 5 (parity), and RAID 10 (mirror + stripe). During maintenance, check RAID status via the controller interface or server management tool: green = OK, amber = degraded (one disk failed), red = failed array. Replace degraded disks promptly using hot-swap procedure and allow rebuild to complete. |
| Uninterruptible Power Supply (UPS) | Provides battery backup power during mains power failure, allowing graceful server shutdown or continued operation. During maintenance, check UPS battery health, load level, and self-test results. Replace batteries on the manufacturer schedule. A failed UPS leaves the server unprotected against power outages. |
| Storage Area Network (SAN) devices | External storage connected to the server via fibre channel or iSCSI. During maintenance, check SAN connectivity, path redundancy, and disk utilisation. Verify that multipath I/O is active and that no single point of failure exists in the storage path. |
| Server LED status indicators | Physical LEDs on the front and rear panels that show system health at a glance. Key indicators include: Power ON/OFF (green = on), HDD activity (blinking green = I/O), fault indicator (amber = warning, red = critical), RAID status (green = healthy, amber = degraded), and network connectivity (green = link up, blinking = traffic). LED blinking patterns are documented in the manufacturer operating manual. |

Standard 42U Server Rack Layout

| Software / OS Maintenance Area | Description |
| --- | --- |
| Server utilisation monitoring | Checking CPU usage, memory usage (physical RAM, virtual, shared), hard disk space (used, free, total), and network usage. High utilisation over sustained periods indicates the server may need a capacity upgrade or load balancing. Monitoring tools and dashboards provide historical trends. |
| Server data backup | Performing full backups (complete copy of all data) and incremental backups (only data changed since last backup). Backup is stored on internal storage, external media, or network-attached storage. After each backup, verify task completion status by checking backup logs for the location, date, backup name, and any errors. Backup must be completed before applying OS patches or performing major maintenance. |
| Server system logs | Three main log types: Security Log (login attempts, access events, policy changes), Application Log (application errors and events), and System Log (OS events, driver failures, service status). Log levels include Information, Warning, and Alert. Log file locations vary by OS (e.g. Event Viewer in Windows, /var/log/ in Linux). Review logs to identify critical errors, hardware failure alerts, OS vulnerability alerts, and service failures. |
| Server antivirus management | Maintaining antivirus software by checking: antivirus update log (pattern/definition currency), antivirus scan log (last scan results), threat log (detected and quarantined threats), and product/licence expiry date. Outdated antivirus patterns leave the server vulnerable to new threats. Schedule regular scans and ensure automatic updates are enabled. |
| Operating System patches and updates | Applying security bulletins and critical patches to the server OS. Check patch availability through Windows Update, WSUS, or the Linux distribution's package manager. Assess each patch for relevance and impact; apply in a test environment first where possible. Schedule patch installation during the maintenance window and document rollback procedures. Reboot the server after patches that require it. |

| Common Fault During Server Maintenance | Cause | Action |
| --- | --- | --- |
| RAID array degraded (amber LED) | One disk in the RAID array has failed or is reporting errors | Identify the failed disk from the RAID controller interface; hot-swap with a compatible replacement disk; monitor rebuild progress and verify RAID returns to healthy status (green LED) |
| Server overheating / thermal shutdown | Dust-blocked vents, failed fan, or server room temperature above 27 °C | Clean dust from all vents and heatsinks; replace failed fans; check server room air conditioning and verify temperature is within 18–27 °C range; check that cable management is not blocking airflow |
| Backup job failed or incomplete | Insufficient storage space on backup media, network interruption during backup, or corrupted backup software configuration | Check backup log for specific error; verify available space on target media; confirm network connectivity; re-run the backup and verify completion status in backup logs |
| Critical OS patch fails to install | Patch conflicts with installed software, insufficient disk space, or service dependency not met | Review the patch installation log for the specific error code; free disk space if needed; resolve software conflicts; retry patch installation; document in maintenance record and escalate if unresolved |
| Antivirus definitions outdated / scan not running | Automatic update disabled, network proxy blocking update server, or licence expired | Check antivirus update settings and proxy configuration; verify licence validity and renew if expired; manually trigger update and scan; verify threat log for any missed detections |
| UPS battery low or failed self-test | Battery degradation over time, excessive load, or environmental temperature too high | Check UPS battery age against manufacturer replacement schedule; reduce load if overloaded; verify server room temperature; replace battery module and re-run self-test to confirm |

Learning Outcome Matrix

| Work Activity | Knowledge Outcome | Performance Outcome | Assessment Method | Evidence Required |
| --- | --- | --- | --- | --- |
| Analyse server maintenance job order | Identify and interpret requirements | Analyse requirements and extract key priorities | Written test on requirement analysis; oral questions on decision criteria | Completed analysis checklist; documented decision rationale |
| Carry out hardware maintenance | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Perform server Operating System maintenance | Understand concepts related to: Perform server Operating System maintenance | Successfully execute: Perform server Operating System maintenance | Practical demonstration and documentation review | Completed task documentation and supervisor sign-off |
| Prepare server maintenance record | Know tools, parts, and materials needed | Gather and organize tools, parts, and materials | Practical observation of tool/material preparation; checklist verification | Photo/log of prepared toolkit; materials checklist |

Practical Exercises

Lab 1: Analyse server maintenance job order

Procedures

- 1. Follow established procedures for analyse server maintenance job order

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of analyse server maintenance job order with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 2: Carry out hardware maintenance

Procedures

- 1. Follow established procedures for carry out hardware maintenance

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of carry out hardware maintenance with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 3: Perform server Operating System maintenance

Procedures

- 1. Follow established procedures for perform server operating system maintenance

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of perform server operating system maintenance with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 4: Prepare server maintenance record

Procedures

- 1. Follow established procedures for prepare server maintenance record

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of prepare server maintenance record with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Employability Skills

This CoCu develops the following employability skills through the work activities:

- Activity: Prepare server maintenance report

- Mapping: Document maintenance activities; record patch installations and security updates; communicate findings and recommendations to management and operations teams

- All activities in enterprise server environment

- Mapping: Coordinate with security and operations teams; schedule maintenance windows; communicate with application owners about planned downtime

- Activities: Identify maintenance requirements, Perform corrective maintenance

- Mapping: Troubleshoot patch installation failures; diagnose backup issues; resolve antivirus and UPS problems using logs and diagnostics

- All preventive and corrective activities

- Mapping: Take responsibility for proactive system health monitoring; make decisions on patch priority and scheduling; work independently on maintenance tasks

- Activities: Identify maintenance requirements, Carry out scheduled preventive maintenance

- Mapping: Plan patch schedules around business needs; organize maintenance windows; coordinate with multiple teams and systems

- All activities in production server environment

- Mapping: Work safely around servers and power distribution; follow change management procedures; manage risk of system downtime during maintenance

- All activities using server monitoring, patching, and backup tools

- Mapping: Use system monitoring tools and event viewers; manage patch installations and rollbacks; configure backup and disaster recovery systems; troubleshoot using logs and diagnostics

- All activities; especially for new server platforms and security threats

- Mapping: Stay informed on critical security updates and vulnerabilities; learn new patching and backup technologies; improve maintenance procedures based on failure analysis

Attitude, Safety and Environmental

- Hot-Swap and Rack Safety: Follow proper hot-swap procedures when replacing drives and power supplies in live servers. Secure heavy equipment to rack rails before releasing; use a server lift or two-person handling for servers above 2U. Ensure rack doors and cable management arms are locked in position to prevent injury.

- Server Room Environment: Maintain server room temperature between 18-27 °C and relative humidity at 40-60 % RH. Monitor environmental sensors continuously and respond immediately to cooling failures. Restrict server room access to authorised personnel and enforce sign-in/sign-out procedures.

- UPS Battery Handling: Wear appropriate PPE (gloves, safety glasses) when inspecting or replacing UPS batteries. Sealed lead-acid and lithium-ion batteries contain hazardous materials; follow manufacturer handling and disposal guidelines. Never short-circuit battery terminals or expose batteries to excessive heat.

- Cable Management for Airflow: Route power, network, and SAN cables through designated cable management trays to maintain unobstructed airflow through server chassis and racks. Label all cables at both ends and avoid blocking front or rear ventilation zones with bundled cables.

- E-waste Management: Properly dispose of defective hardware components, circuit boards, and packaging materials through authorised e-waste recycling centres. Never dispose of electronics in regular trash or landfill.

- Energy Efficiency: Configure systems to use power-saving modes and efficient power supplies. Avoid unnecessary idle time and follow organisational energy conservation policies.

- Punctuality and Reliability: Complete server maintenance work on schedule to meet user handover deadlines and support business operations.

- Integrity in Documentation: Prepare accurate and complete server maintenance reports with all hardware details, software versions, and test results. Honest documentation supports future troubleshooting and maintains system integrity.

- Teamwork and Support: Collaborate with help desk staff and other technicians; share knowledge about system configurations and troubleshooting techniques to improve overall technical capability.

- Attention to Detail: Verify every connection, configuration setting, and test result to ensure server maintenance tasks are completed correctly and systems function as intended.

References

# CoCu 6: Computer Network Connectivity Set-up (120 hrs)

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION |  |
| --- | --- | --- |
| LEVEL | L3 |  |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 6: Computer Network Connectivity Set-up |  |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE COMPUTER NETWORK CONFIGURATION SPECIFICATION · CARRY OUT COMPUTER NETWORK CONFIGURATION · PERFORM COMPUTER NETWORK CONNECTIVITY TEST · CARRY OUT COMPUTER NETWORK TROUBLESHOOT · PREPARE COMPUTER NETWORK CONNECTIVITY REPORT |  |
| NO. CODE | IT-020-3:2013 - CoCu 6 / P(6/7) |  |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 6: Computer Network Connectivity Set-up (Persediaan Sambungan Rangkaian Komputer) | CoCu 6: Computer Network Connectivity Set-up (Persediaan Sambungan Rangkaian Komputer) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE COMPUTER NETWORK CONFIGURATION SPECIFICATION · CARRY OUT COMPUTER NETWORK CONFIGURATION · PERFORM COMPUTER NETWORK CONNECTIVITY TEST · CARRY OUT COMPUTER NETWORK TROUBLESHOOT · PREPARE COMPUTER NETWORK CONNECTIVITY REPORT | ANALYSE COMPUTER NETWORK CONFIGURATION SPECIFICATION · CARRY OUT COMPUTER NETWORK CONFIGURATION · PERFORM COMPUTER NETWORK CONNECTIVITY TEST · CARRY OUT COMPUTER NETWORK TROUBLESHOOT · PREPARE COMPUTER NETWORK CONNECTIVITY REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 6 / P(6/7) | PAGE: 133 - 138 |

| SET-UP CONTEXT | WIRED LAN / NEW OFFICE SET-UP | WIRELESS / EXTENSION TO EXISTING NETWORK |
| --- | --- | --- |
| Job order | New wired LAN installation for an office floor. Configuration specification includes number of drops, cable type (Cat6), IP scheme (static or DHCP), subnet mask, gateway, DNS, switch port assignments, and VLAN if required. Equipment list: switch, patch panel, patch cords, RJ45 faceplates. | Adding wireless access points to extend coverage in an existing network. Specification includes SSID, security (WPA2/WPA3), channel allocation, access point placement, and authentication method. Existing IP scheme (DHCP) and VLAN are reused or extended. |
| Tools and parts | Crimping tool, cable tester, punch-down tool, screwdriver set, cable ties, label maker. Parts: UTP Cat6 cable reels, RJ45 connectors, patch panel, faceplates, managed switch, patch cords. | Wireless site survey tool (app or hardware), cable tester, screwdriver set, PoE injector or PoE switch. Parts: access points, mounting brackets, Cat6 patch cords, PoE cables. |
| Report | Network connectivity report with IP scheme, switch port map, cable test results, ping and traceroute test results, UAT sign-off, and network diagram. | Connectivity report with access point placement map, SSID configuration, wireless coverage test results (signal strength per area), authentication test results, and UAT sign-off. |

| Type of Tools and Equipment | Description |
| --- | --- |
| Crimping tool and RJ45 connectors | Used to terminate UTP cable with RJ45 connectors. The crimping tool presses the connector contacts onto the cable wires. Correct wire order (T568A or T568B) must be followed; inconsistent termination causes connectivity failures. Always test the cable after crimping. |
| Cable tester / LAN tester | Used to verify that each wire pair in a UTP or patch cable is correctly terminated and can carry signal. Tests for open circuits, short circuits, crossed pairs, and split pairs. Essential for every new cable run and for troubleshooting existing connections. |
| Punch-down tool | Used to terminate cable wires onto patch panels and keystone jacks (faceplates). The tool seats the wire into the IDC (insulation displacement contact) and trims the excess. Correct colour code (T568A or T568B) must match the patch panel and faceplate. |
| Managed network switch | The central device that connects all network drops. Managed switches allow VLAN configuration, port speed settings, and monitoring. During set-up, configure port assignments, enable or disable ports, and set trunk links to other switches or the router. |
| Router / gateway | Connects the local network to the WAN or internet. Configured with the gateway IP address, NAT, DHCP scope (if used), and firewall rules. The router is the default gateway for all devices on the network. |
| Wireless access point (AP) | Provides Wi-Fi connectivity for wireless devices. Configured with SSID, security protocol (WPA2/WPA3), channel, and transmit power. Placement affects coverage and signal quality; a wireless site survey helps determine optimal positions. PoE (Power over Ethernet) simplifies installation by delivering power and data over one cable. |
| Label maker and cable tags | Used to label every cable run, patch panel port, faceplate, and switch port with a unique identifier. Labels match the network documentation and make troubleshooting faster. Labelling must be done during installation, not after. |

| % | Hrs | Work activity | Knowledge 30% | Performance 70% | Total |
| --- | --- | --- | --- | --- | --- |
| 20% | 24 | Analyse computer network configuration specification | 7.2 | 16.8 | 24.0 |
| 25% | 30 | Carry out computer network configuration | 9.0 | 21.0 | 30.0 |
| 20% | 24 | Perform computer network connectivity test | 7.2 | 16.8 | 24.0 |
| 25% | 30 | Carry out computer network troubleshoot | 9.0 | 21.0 | 30.0 |
| 10% | 12 | Prepare computer network connectivity report | 3.6 | 8.4 | 12.0 |
| 100% | 120 |  | 36.0 | 84.0 | 120.0 |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 6: Computer Network Connectivity Set-up (Persediaan Sambungan Rangkaian Komputer) | CoCu 6: Computer Network Connectivity Set-up (Persediaan Sambungan Rangkaian Komputer) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE COMPUTER NETWORK CONFIGURATION SPECIFICATION · CARRY OUT COMPUTER NETWORK CONFIGURATION · PERFORM COMPUTER NETWORK CONNECTIVITY TEST · CARRY OUT COMPUTER NETWORK TROUBLESHOOT · PREPARE COMPUTER NETWORK CONNECTIVITY REPORT | ANALYSE COMPUTER NETWORK CONFIGURATION SPECIFICATION · CARRY OUT COMPUTER NETWORK CONFIGURATION · PERFORM COMPUTER NETWORK CONNECTIVITY TEST · CARRY OUT COMPUTER NETWORK TROUBLESHOOT · PREPARE COMPUTER NETWORK CONNECTIVITY REPORT |
| NO. CODE | IT-020-3:2013 - CoCu 6 / P(6/7) | PAGE: 139 - 144 |

| Hardware Component | Description |
| --- | --- |
| Network Interface Card (NIC) | The adapter that connects a computer or peripheral to the network. NICs may be built into the motherboard (onboard) or added as an expansion card. Each NIC has a unique MAC address. During set-up, install the NIC driver, configure the IP address (static or DHCP), subnet mask, gateway, and DNS. Check the NIC LED: green steady = link up, blinking = traffic, no light = no connection. |
| UTP cable (Cat5e / Cat6 / Cat6a) | Unshielded twisted pair cable used for Ethernet LAN connections. Cat5e supports up to 1 Gbps at 100 m; Cat6 supports up to 10 Gbps at 55 m; Cat6a supports 10 Gbps at 100 m. Straight-through wiring (T568B both ends) is standard for PC-to-switch connections. Crossover wiring is used for like-to-like connections, though most modern devices support auto-MDI/MDIX. |
| Fibre optic cable (single-mode / multi-mode) | Used for long-distance runs or high-speed backbone connections between switches, floors, or buildings. Single-mode uses a narrow core for distances up to 40 km; multi-mode uses a wider core for up to 550 m. Connector types include LC, SC, and ST — they must match the switch transceiver or SFP module. Handle with care; do not bend below the minimum bend radius. |
| Patch panel | A passive device mounted in the network rack that terminates all cable runs from wall faceplates. Each port on the patch panel connects to a switch port via a short patch cord. Patch panels organise cabling, simplify moves and changes, and reduce wear on switch ports. Label every port to match the faceplate and switch port. |
| RJ45 faceplate and keystone jack | Mounted on the wall at each network drop location. The keystone jack terminates the cable from the patch panel; the faceplate provides a neat, accessible connection point for the user's patch cord. Wire colour code must match the patch panel (T568A or T568B). |
| Main Distribution Frame (MDF) and rack | The central location where all network cabling terminates and active equipment (switches, routers, patch panels, UPS) is housed. The rack provides organised mounting; cable management trays and power distribution units (PDUs) keep the installation tidy and maintainable. Environmental control (ventilation or air conditioning) prevents overheating. |

Common Network Topologies

| Software / Configuration Area | Description |
| --- | --- |
| IP addressing and subnetting | Assigning IP addresses to devices using a static scheme or DHCP server. The subnet mask defines the network boundary; the gateway is the router's IP for traffic outside the subnet; DNS servers resolve domain names. Common private ranges: 192.168.x.x, 10.x.x.x, 172.16-31.x.x. Plan the IP scheme before configuration to avoid conflicts. |
| VLAN configuration | Virtual LANs segment the physical network into logical groups (e.g. by department, function, or security level). VLANs are configured on managed switches by assigning ports to VLAN IDs. Trunk ports carry traffic for multiple VLANs between switches. VLAN tagging follows IEEE 802.1Q. VLANs improve security and reduce broadcast traffic. |
| Network connectivity testing tools | Standard tools for verifying connectivity: ping (tests reachability to gateway, DNS, and external hosts), tracert / traceroute (shows the path packets take and identifies where delays or failures occur), ipconfig / ifconfig (displays current IP configuration), and browser test (confirms internet access). Print and scan tests verify peripheral connectivity. All results are documented in the connectivity report. |
| Network security configuration | Basic security settings applied during network set-up: setting the switch management password, disabling unused ports, enabling port security (MAC address limiting), and configuring wireless security (WPA2/WPA3 with strong passphrase). These settings protect the new network from unauthorised access before it is handed over to the user. |

RJ45 Cable Wiring Standards (T568A / T568B)

| Common Fault During Network Set-up | Cause | Action |
| --- | --- | --- |
| No link light on NIC or switch port | Cable not connected, faulty cable termination, NIC not seated, or switch port disabled | Check cable connections at both ends; test cable with LAN tester; reseat NIC; verify switch port is enabled in management interface |
| Ping to gateway fails (no connectivity) | Wrong IP address, subnet mask, or gateway; NIC driver not installed; cable fault | Run ipconfig to verify settings; compare with the configuration specification; reinstall NIC driver; test with a known-good cable |
| Can ping gateway but not external hosts | DNS misconfigured, router/firewall blocking traffic, or ISP issue | Check DNS settings; try pinging an external IP (e.g. 8.8.8.8) to isolate DNS vs routing; check router NAT and firewall rules |
| Wireless device connects but no internet | Wrong SSID security key, DHCP scope exhausted, AP channel interference, or VLAN mismatch | Verify SSID and security settings; check DHCP lease pool; change AP channel to avoid interference; confirm VLAN assignment matches the wired network |
| Intermittent connection drops | Damaged cable, electromagnetic interference, faulty NIC, or switch port flapping | Replace suspect cable; relocate cable away from power lines or fluorescent lights; test NIC in another slot; check switch port error counters and logs |
| Print or scan test fails over network | Printer/scanner not on same subnet, firewall blocking port, or driver not installed | Verify printer/scanner IP is on the correct subnet; check Windows Firewall or security software; install correct print/scan driver and configure port |

Learning Outcome Matrix

| Work Activity | Knowledge Outcome | Performance Outcome | Assessment Method | Evidence Required |
| --- | --- | --- | --- | --- |
| Analyse computer network configuration specification | Identify and interpret requirements | Analyse requirements and extract key priorities | Written test on requirement analysis; oral questions on decision criteria | Completed analysis checklist; documented decision rationale |
| Carry out computer network configuration | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Perform computer network connectivity test | Know testing procedures and acceptance criteria | Execute tests and document results | Practical test execution; result documentation | Test results log; signed-off test report |
| Carry out computer network troubleshoot | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Prepare computer network connectivity report | Know tools, parts, and materials needed | Gather and organize tools, parts, and materials | Practical observation of tool/material preparation; checklist verification | Photo/log of prepared toolkit; materials checklist |

Practical Exercises

Lab 1: Analyse computer network configuration specification

Procedures

- 1. Follow established procedures for analyse computer network configuration specification

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of analyse computer network configuration specification with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 2: Carry out computer network configuration

Procedures

- 1. Follow established procedures for carry out computer network configuration

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of carry out computer network configuration with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 3: Perform computer network connectivity test

Procedures

- 1. Follow established procedures for perform computer network connectivity test

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of perform computer network connectivity test with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 4: Carry out computer network troubleshoot

Procedures

- 1. Follow established procedures for carry out computer network troubleshoot

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of carry out computer network troubleshoot with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 5: Prepare computer network connectivity report

Procedures

- 1. Follow established procedures for prepare computer network connectivity report

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of prepare computer network connectivity report with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Employability Skills

This CoCu develops the following employability skills through the work activities:

- Activity: Prepare network connectivity report

- Mapping: Document configuration settings and test results; explain network setup to users; communicate connectivity issues and recommendations

- All activities in shared network environment

- Mapping: Coordinate with network managers on IP and VLAN allocation; work with switch/router administrators; support end-users with connectivity troubleshooting

- All activities involving diagnosis and testing

- Mapping: Troubleshoot connectivity failures; isolate hardware vs configuration vs external issues; diagnose DNS, VLAN, and wireless problems

- Cable installation and configuration activities

- Mapping: Work independently on cabling and IP configuration; take responsibility for proper cable termination and testing; verify connectivity before handover

- Cable planning and installation

- Mapping: Plan cable routes and termination points; organize patch panels logically; document IP addressing scheme and VLAN assignments

- Cable handling and MDF work

- Mapping: Handle cables properly (avoid min bend radius); work safely in network infrastructure; follow proper cable management practices; manage time to meet connectivity deadlines

- All activities using network hardware and configuration tools

- Mapping: Install NICs and configure IP settings; terminate cables to T568A/B standard; use LAN testers and network diagnostic tools; configure VLANs and wireless security

- All activities; especially new cabling standards and network technologies

- Mapping: Stay current on cabling standards (Cat5e/6/6a); learn new VLAN and wireless configuration methods; improve troubleshooting efficiency

Attitude, Safety and Environmental

- Cable Management and Trip Hazards: Route all network and power cables through cable trays, conduits, or floor ducts to prevent trip hazards in walkways and workspaces. Avoid bundling excessive cables together, as tightly packed bundles restrict airflow and increase fire risk. Secure loose cables immediately during installation.

- Fibre Optic Safety: Never look directly into the end of a fibre optic cable or connector, as laser light can cause permanent eye damage. Wear safety glasses when cleaving or splicing fibre; handle bare fibre carefully to avoid glass splinter injuries. Dispose of fibre offcuts in a designated sharps container, not in general waste.

- Working at Height: Use approved ladders, scaffolding, or access platforms when running cables through ceiling voids or elevated cable trays. Follow organisational working-at-height procedures, including the use of a spotter or buddy system. Inspect equipment before use and never overreach.

- Electrical Safety with PoE and Crimping Tools: Exercise caution when working with Power over Ethernet (PoE) equipment, as PoE switches deliver up to 90 W per port. Disconnect cables from PoE ports before crimping or terminating. Use crimping and punch-down tools correctly to avoid hand injuries; keep cutting blades away from fingers and replace dull blades promptly.

- E-waste Management: Properly dispose of defective hardware components, circuit boards, and packaging materials through authorised e-waste recycling centres. Never dispose of electronics in regular trash or landfill.

- Energy Efficiency: Configure systems to use power-saving modes and efficient power supplies. Avoid unnecessary idle time and follow organisational energy conservation policies.

- Punctuality and Reliability: Complete network connectivity set-up work on schedule to meet user handover deadlines and support business operations.

- Integrity in Documentation: Prepare accurate and complete network connectivity reports with all hardware details, software versions, and test results. Honest documentation supports future troubleshooting and maintains system integrity.

- Teamwork and Support: Collaborate with help desk staff and other technicians; share knowledge about system configurations and troubleshooting techniques to improve overall technical capability.

- Attention to Detail: Verify every connection, configuration setting, and test result to ensure network connectivity is correctly set up and systems function as intended.

References

# CoCu 7: Mobile Device Configuration (60 hrs)

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION |  |
| --- | --- | --- |
| LEVEL | L3 |  |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 7: Mobile Device Configuration |  |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB ORDER/CHANGE REQUEST · CARRY OUT MOBILE DEVICE CONFIGURATION · PERFORM MOBILE DEVICE TROUBLESHOOT · CARRY OUT MOBILE DEVICE COMMISSIONING |  |
| NO. CODE | IT-020-3:2013 - CoCu 7 / P(7/7) |  |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 7: Mobile Device Configuration (Konfigurasi Peranti Mudah Alih) | CoCu 7: Mobile Device Configuration (Konfigurasi Peranti Mudah Alih) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB ORDER/CHANGE REQUEST · CARRY OUT MOBILE DEVICE CONFIGURATION · PERFORM MOBILE DEVICE TROUBLESHOOT · CARRY OUT MOBILE DEVICE COMMISSIONING | ANALYSE JOB ORDER/CHANGE REQUEST · CARRY OUT MOBILE DEVICE CONFIGURATION · PERFORM MOBILE DEVICE TROUBLESHOOT · CARRY OUT MOBILE DEVICE COMMISSIONING |
| NO. CODE | IT-020-3:2013 - CoCu 7 / P(7/7) | PAGE: 145 - 150 |

| SET-UP CONTEXT | CORPORATE SMARTPHONE / NEW USER | FIELD TABLET / REPLACEMENT DEVICE |
| --- | --- | --- |
| Job order | New employee onboarding: configure a company smartphone (Android or iOS) with corporate Wi-Fi (SSID, WPA2/WPA3), email (Exchange or IMAP), VPN client and profile, security certificates, screen lock (PIN or biometric), and encryption. Install required apps per organisational policy. MDM enrolment if applicable. | Replace a damaged or outdated tablet used for field work. Transfer user data from backup, configure Wi-Fi and cellular data, install field applications, configure GPS and location services, and apply company security policy. Verify device specification matches minimum requirements (RAM, storage, OS version). |
| Tools and parts | Charging cable, SIM tool, Wi-Fi access for setup, MDM enrolment QR code or URL, company email credentials, VPN profile file or configuration details, list of required apps with licence keys. | Replacement tablet, charging cable, SIM card (if cellular), backup media or cloud account credentials for data restore, field application installer files or app store access, protective case for field use. |
| Report | Commissioning report with asset tag, device serial number, IMEI, user name, department, configured services (email, VPN, Wi-Fi), installed apps, MDM enrolment status, and user acceptance sign-off. | Commissioning report with old and new asset tags, data migration confirmation, device specification, configured services, application list, security settings applied, and user acceptance sign-off. Company inventory updated. |

| Type of Tools and Equipment | Description |
| --- | --- |
| Mobile Device Management (MDM) software | Enterprise software (e.g. Microsoft Intune, VMware Workspace ONE, Jamf) used to remotely configure, monitor, and secure mobile devices at scale. MDM pushes Wi-Fi profiles, email settings, VPN configurations, security policies (encryption, screen lock, remote wipe), and app installations to enrolled devices. Essential for organisations managing more than a handful of mobile devices. |
| SIM ejector tool and SIM cards | The SIM ejector tool opens the SIM tray on smartphones and tablets. SIM cards (nano, micro, or eSIM) provide cellular connectivity. During configuration, insert the correct SIM and verify cellular data connectivity, carrier settings, and APN configuration. For eSIM, scan the carrier QR code or enter the activation details. |
| USB cable and charging adapter | Used to connect the mobile device to a computer for initial setup, data transfer, firmware update, or charging. USB-C is standard on most modern devices; Lightning is used on older iPhones. Ensure the cable supports data transfer, not charge-only. |
| Wi-Fi network access | Required during initial setup to download updates, enrol in MDM, install apps, and configure cloud accounts. The technician must have the SSID, security key, and any certificate or 802.1X credentials needed to connect the device to the corporate Wi-Fi. |
| Backup and restore tools | Built-in tools (e.g. Google Backup, iCloud, Samsung Smart Switch) or third-party software used to back up user data before troubleshooting and to restore data onto replacement devices. Backup includes contacts, photos, app data, messages, and settings. Verify backup completeness before performing a factory reset or device swap. |
| Diagnostic and testing apps | Tools used to verify device functionality after configuration: Wi-Fi analyser (signal strength and channel), speed test (bandwidth verification), email test (send/receive), VPN connection test, and GPS accuracy test. These confirm the device meets the job order requirements before commissioning. |

| % | Hrs | Work activity | Knowledge 30% | Performance 70% | Total |
| --- | --- | --- | --- | --- | --- |
| 25% | 15 | Analyse job order/change request | 4.5 | 10.5 | 15.0 |
| 35% | 21 | Carry out mobile device configuration | 6.3 | 14.7 | 21.0 |
| 30% | 18 | Perform mobile device troubleshoot | 5.4 | 12.6 | 18.0 |
| 10% | 6 | Carry out mobile device commissioning | 1.8 | 4.2 | 6.0 |
| 100% | 60 |  | 18.0 | 42.0 | 60.0 |

| PROGRAM CODE AND NAME | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) | IT-020-3:2013 COMPUTER SYSTEM OPERATION (Operasi Sistem Komputer) |
| --- | --- | --- |
| LEVEL | L3 | L3 |
| NO. AND UNIT TITLE OF COMPETENCY | CoCu 7: Mobile Device Configuration (Konfigurasi Peranti Mudah Alih) | CoCu 7: Mobile Device Configuration (Konfigurasi Peranti Mudah Alih) |
| NO. AND WORK ACTIVITY STATEMENT | ANALYSE JOB ORDER/CHANGE REQUEST · CARRY OUT MOBILE DEVICE CONFIGURATION · PERFORM MOBILE DEVICE TROUBLESHOOT · CARRY OUT MOBILE DEVICE COMMISSIONING | ANALYSE JOB ORDER/CHANGE REQUEST · CARRY OUT MOBILE DEVICE CONFIGURATION · PERFORM MOBILE DEVICE TROUBLESHOOT · CARRY OUT MOBILE DEVICE COMMISSIONING |
| NO. CODE | IT-020-3:2013 - CoCu 7 / P(7/7) | PAGE: 151 - 154 |

| Hardware / Device Component | Description |
| --- | --- |
| Smartphone (Android / iOS) | A mobile phone with a full operating system capable of running corporate applications. Key specifications to assess: maker/brand/model, RAM capacity, storage size, screen display type, input method (touchscreen), connectivity (Wi-Fi, Bluetooth, NFC, cellular), OS version, and IMEI. The device must meet the minimum hardware and software requirements specified in the job order for all required applications. |
| Tablet (Android / iOS / Windows) | A larger-screen mobile device used for field work, meetings, presentations, or kiosk applications. Configuration is similar to a smartphone but may include stylus input, keyboard accessories, and larger storage for field data. Cellular-enabled tablets require a SIM card or eSIM for mobile data. Some organisations use tablets with rugged cases for outdoor or industrial use. |
| Laptop (portable computer) | A portable computer running Windows, macOS, or Linux. Configuration includes OS setup, network (Wi-Fi and wired), VPN, email, applications, and security (BitLocker or FileVault encryption, screen lock). Asset tagging and user handover documentation are part of commissioning. Laptops bridge the gap between desktop and mobile; they follow both desktop build and mobile device configuration procedures. |
| Connectivity interfaces | The connection types available on mobile devices: Wi-Fi (802.11ac/ax for wireless LAN), Cellular (4G/5G for mobile data), Bluetooth (short-range for peripherals like headsets, keyboards), NFC (near-field communication for contactless pairing or payment), Infrared (legacy, rare on modern devices), and USB (data transfer and charging). Each must be configured or disabled per the organisational security policy. |
| Mobile device security features | Built-in security capabilities: screen lock (PIN, pattern, fingerprint, face recognition), device encryption (full-disk encryption enabled by default on modern devices), remote wipe (erase device data remotely if lost or stolen, via MDM or Find My Device), certificate store (for corporate Wi-Fi 802.1X and VPN authentication), and app permissions (control which apps can access camera, microphone, location, contacts). |

| Software / Configuration Area | Description |
| --- | --- |
| Network and connectivity configuration | Setting up Wi-Fi (SSID, security key, 802.1X if enterprise), cellular APN, Bluetooth pairing, and VPN client. IP addressing is typically DHCP for mobile devices. Authentication may use username/password, certificate, or both. Test each connection type after configuration: Wi-Fi signal strength, cellular data speed, VPN tunnel establishment, and Bluetooth peripheral pairing. |
| Email and collaboration setup | Configuring corporate email using Exchange ActiveSync, IMAP, or POP as specified in the job order. Settings include server address, port, SSL/TLS, username, and password or OAuth. Calendar and contacts sync are configured alongside email. Collaboration apps (e.g. Microsoft Teams, Google Workspace) are installed and signed in. Test by sending and receiving a test email and verifying calendar sync. |
| Mobile application management | Installing, configuring, and updating required applications per the job order. Apps may be deployed through an enterprise app store, MDM push, or manual installation. Verify software licences, version compatibility with the device OS, and minimum device requirements (RAM, storage). Configure app-specific settings (e.g. default browser, offline mode for field apps). Remove or restrict personal apps if required by company policy. |
| Security policy enforcement | Applying the organisational security policy to the device: enable device encryption, set screen lock timeout, enforce password complexity, enable remote wipe capability, restrict USB debugging, disable installation from unknown sources, and configure automatic OS and app updates. On MDM-managed devices, these policies are pushed automatically. On manually configured devices, each setting must be applied and documented individually. |

OSI Model vs TCP/IP Model

| Common Fault During Mobile Configuration | Cause | Action |
| --- | --- | --- |
| Wi-Fi connects but no internet access | Wrong security key, DHCP scope exhausted, MAC filtering blocking device, or proxy misconfigured | Verify SSID and security key; check DHCP server lease pool; add device MAC to allowed list if filtering is enabled; check proxy settings in Wi-Fi advanced options |
| Email fails to sync (authentication error) | Wrong server address or port, incorrect credentials, security certificate not trusted, or two-factor authentication not configured | Verify server settings against the job order specification; re-enter credentials; install required security certificate; configure app-specific password if 2FA is enabled |
| VPN connection fails | VPN profile incorrect, server address wrong, certificate expired or missing, or firewall blocking VPN port | Verify VPN configuration (server, protocol, port); reinstall VPN certificate; check with network team if VPN server is reachable; try alternative VPN protocol if available |
| App installation fails (incompatible) | Device OS version too old, insufficient storage, app requires higher RAM, or app not available in region | Check app minimum requirements against device specification; update device OS if possible; free storage space; download app from alternative source if region-restricted |
| Device not enrolling in MDM | Enrolment URL or QR code expired, device OS not supported by MDM, or network blocking MDM server | Verify enrolment link is current; check device OS version against MDM requirements; connect to a network that allows access to the MDM server; contact MDM administrator |
| Battery drains rapidly after configuration | Background sync too frequent, GPS always on, screen brightness too high, or rogue app consuming resources | Adjust email sync frequency; set GPS to battery-saving mode; enable auto-brightness; check battery usage in settings to identify the draining app; uninstall or restrict it |

Learning Outcome Matrix

| Work Activity | Knowledge Outcome | Performance Outcome | Assessment Method | Evidence Required |
| --- | --- | --- | --- | --- |
| Analyse job order/change request | Identify and interpret requirements | Analyse requirements and extract key priorities | Written test on requirement analysis; oral questions on decision criteria | Completed analysis checklist; documented decision rationale |
| Carry out mobile device configuration | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |
| Perform mobile device troubleshoot | Understand concepts related to: Perform mobile device troubleshoot | Successfully execute: Perform mobile device troubleshoot | Practical demonstration and documentation review | Completed task documentation and supervisor sign-off |
| Carry out mobile device commissioning | Know procedures and troubleshooting techniques | Execute procedures and resolve issues | Practical demonstration; problem-solving observed | Completed task log; troubleshooting notes; final working state |

Practical Exercises

Lab 1: Analyse job order/change request

Procedures

- 1. Follow established procedures for analyse job order/change request

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of analyse job order/change request with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 2: Carry out mobile device configuration

Procedures

- 1. Follow established procedures for carry out mobile device configuration

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of carry out mobile device configuration with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 3: Perform mobile device troubleshoot

Procedures

- 1. Follow established procedures for perform mobile device troubleshoot

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of perform mobile device troubleshoot with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Lab 4: Carry out mobile device commissioning

Procedures

- 1. Follow established procedures for carry out mobile device commissioning

- 2. Complete the activity according to job specifications

- 3. Verify the work meets quality standards

- 4. Document the completion and any issues encountered

Expected Outcome

Successful completion of carry out mobile device commissioning with no errors

Assessment Checklist

- Activity completed according to procedure

- Quality standards met

- No safety incidents

- Documentation complete

Employability Skills

This CoCu develops the following employability skills through the work activities:

- Activity: Prepare mobile device configuration report

- Mapping: Document device setup, security policies, and user handover instructions; explain security and connectivity features to mobile device users

- All configuration activities in corporate mobile environment

- Mapping: Coordinate with MDM, network, and security teams; support end-users with device provisioning; share knowledge about new devices and OS versions

- All activities involving connectivity and app setup

- Mapping: Troubleshoot Wi-Fi, email, and VPN authentication failures; diagnose app compatibility and installation issues; resolve battery drain and performance problems

- All device configuration activities

- Mapping: Work independently on mobile device provisioning; make decisions on security policy enforcement; manage user expectations for device capabilities

- Activities: Assess mobile configuration requirements, Prepare devices for deployment

- Mapping: Plan configuration sequence (network, email, apps, security); organize device setup and user handover; coordinate with MDM push schedules

- All activities involving sensitive data and security

- Mapping: Follow security policies for device encryption and remote wipe; manage time to complete provisioning efficiently; maintain confidentiality of credentials

- All activities using mobile OS, connectivity, and MDM tools

- Mapping: Configure Wi-Fi, cellular, VPN, and Bluetooth connectivity; set up email, calendar, and collaboration apps; enforce mobile device security policies

- All activities; especially for new device models and OS versions

- Mapping: Stay current on new mobile device features and security requirements; learn new app deployment and MDM enrollment methods; improve provisioning procedures

Attitude, Safety and Environmental

- Lithium Battery Safety: Handle mobile device batteries with care; do not puncture, bend, or expose them to excessive heat. A swollen or damaged lithium battery poses fire and chemical burn risks -- discontinue use immediately, place the device on a non-flammable surface, and follow organisational hazardous material procedures for disposal. Never attempt to pry open a swollen battery.

- Screen and Glass Handling: Mobile device screens are made of fragile glass that can crack or shatter during repair or configuration. Wear safety glasses when working with cracked screens and handle broken devices with gloves to avoid cuts. Dispose of broken screen glass in a designated sharps or e-waste container.

- Data Privacy and Security: Perform a complete data wipe (factory reset) before redeploying or disposing of any mobile device to protect personal and corporate data. Verify the wipe is complete by checking that no user accounts, files, or cached data remain. Follow the organisation's data protection policy (e.g. PDPA) when handling devices that contain personal information.

- Ergonomic Device Testing: Maintain a neutral wrist and neck posture when testing and configuring mobile devices for extended periods. Use a device stand or holder at eye level to reduce repetitive strain. Take regular breaks during batch provisioning sessions to prevent fatigue-related errors and musculoskeletal discomfort.

- E-waste Management: Properly dispose of defective hardware components, circuit boards, and packaging materials through authorised e-waste recycling centres. Never dispose of electronics in regular trash or landfill.

- Energy Efficiency: Configure systems to use power-saving modes and efficient power supplies. Avoid unnecessary idle time and follow organisational energy conservation policies.

- Punctuality and Reliability: Complete mobile device configuration work on schedule to meet user handover deadlines and support business operations.

- Integrity in Documentation: Prepare accurate and complete commissioning reports with all hardware details, software versions, and test results. Honest documentation supports future troubleshooting and maintains system integrity.

- Teamwork and Support: Collaborate with help desk staff and other technicians; share knowledge about system configurations and troubleshooting techniques to improve overall technical capability.

- Attention to Detail: Verify every connection, configuration setting, and test result to ensure mobile device configuration tasks are completed correctly and devices function as intended.

References